Boot FST

Module Design Specification

Version: 1.1.1RC                                                       Status: Draft

Date: 07-January-2021

Document Author: Amarnath C

Document List of Approvers:

Firmware Architect: Rahil Malhotra

Validation Architect: Mahaveer Addanki

Functional Safety Architect: Srikanth Kaniyanoor Srinivasan

Functional Safety Manager: Francesco Pingitore

Applicable Standard: ISO 26262 and IEC 61508

Template Version: FuSa_Documentation_Word_Template.docx v0.5

FuSa Lifecycle Phase: FS2

Intel Top Secret

You may not use or facilitate the use of this document in connection with any infringement or other legal analysis concerning Intel products described herein. You agree to grant Intel a non-exclusive, royalty-free license to any patent claim thereafter drafted which includes subject matter disclosed herein

No license (express or implied, by estoppel or otherwise) to any intellectual property rights is granted by this document.

All information provided here is subject to change without notice. Contact your Intel representative to obtain the latest Intel product specifications and roadmaps.

The products described may contain design defects or errors known as errata which may cause the product to deviate from published specifications. Current characterized errata are available on request.

Copies of documents which have an order number and are referenced in this document may be obtained by calling 1-800-548-4725 or by visiting:  http://www.intel.com/design/literature.htm

Intel technologies’ features and benefits depend on system configuration and may require enabled hardware, software or service activation. Learn more at http://www.intel.com/ or from the OEM or retailer.

Intel and the Intel logo are trademarks of Intel Corporation in the U.S. and/or other countries.

*Other names and brands may be claimed as the property of others.

Copyright © 2021, Intel Corporation. All rights reserved.

Contents

1.0	Introduction	9

1.1	Purpose	9

1.2	Audience	9

1.3	Acronyms and Terminology	9

1.4	Reference Documents	10

2.0	Document Scope	12

2.1	Prerequisite Documents	12

2.2	In Scope	12

2.3	Out of Scope	12

2.4	Known Gaps/Opens	12

2.5	ISO26262 and IEC61508 Fulfillment Matrix	12

2.5.1	ISO26262 Fulfillment Matrix	12

2.5.2	IEC61508 Fulfillment Matrix	13

3.0	Global Design Decisions	14

4.0	Functional Description	15

4.1	Overview	15

4.2	Type Definitions	16

4.2.1	Local type definitions	16

4.3	Configuration Parameters	24

4.4	API Functions	25

4.4.1	Boot FST initialization	25

4.4.2	Boot FST stop	26

4.4.3	Boot FST application	28

4.5	Interrupt Handler	31

4.6	Local functions	31

4.6.1	Boot FST timer expiry callback function	31

4.6.2	Boot FST bios boot callback function	32

4.6.3	Boot FST POSC results callback function	34

4.6.4	Boot FST STL results callback function	36

4.6.5	Boot FST ODCC SS callback function	38

4.6.6	Boot FST override config param callback function	40

4.6.7	Clock monitor Error Injection(EI) test	42

4.6.8	Clock monitor pre-mask status check	45

4.6.9	Clock monitor count done status check	46

4.6.10	Clock monitor PPM range status check	48

4.6.11	Clock monitor count error injection test	50

4.6.12	Clock monitor lock error injection test	54

4.6.13	Clock monitor dead clock error injection test	56

4.6.14	Root parity Error Injection(EI) test	59

4.6.15	ROM ECC error injection test	62

4.6.16	Boot FST frCPU check	63

4.6.17	NVIC interrupt priority register check	65

4.6.18	Async commands register	66

4.6.19	Async commands de-register	68

4.6.20	Host boot monitoring	70

4.6.21	Host BIOS boot monitor	73

4.6.22	POSC results packet validate	75

4.6.23	POSC – get number of cores enabled	77

4.6.24	POSC results respond	79

4.6.25	POSC test results monitor	80

4.6.26	RAVDM test	82

4.6.27	RAVDM ping test	83

4.6.28	RAVDM ITD parameters fetch	86

4.6.29	PMC IPC test	88

4.6.30	ODCC test – Dummy snapshot compare	91

4.6.31	Override config param data set	93

4.6.32	Override config param handle	95

4.6.33	STL results packet validation	96

4.6.34	Uncore STL execution check	98

4.6.35	Core STL execution check	100

4.6.36	System time set	101

4.6.37	STL results respond	103

4.6.38	Host startup STL results monitor	104

4.7	Imported functions	106

4.7.1	GP timer expiry callback function	106

4.7.2	Periodic timer expiry callback function	108

4.7.3	RTOS timer error handler	109

4.7.4	Timer Error Injection(EI) test	110

4.7.5	WDT alarm mask and wdt start	113

4.7.6	WDT configure and start	114

4.7.7	WDT difference error handle	116

4.7.8	WDT error handler	117

4.7.9	WDT NOK status error handle	118

4.7.10	WDT test	119

4.7.11	GP timer difference calculation	122

4.7.12	GP timer test	123

4.7.13	Periodic timer difference calculation	126

4.7.14	Periodic timer test	127

4.7.15	FMM Scratchpad register test	129

4.7.16	Multiplier and Divider check	131

4.7.17	Bus Matrix check	134

4.7.18	MPU and NVIC registers read	138

4.8	Imported Types	139

5.0	Data Description	141

Figures

Figure 1: Boot FST initialization flow	27

Figure 2: Boot FST stop flow	29

Figure 3: Boot FST app flow	31

Figure 4: Boot fst Timer expiry callback function flow	33

Figure 5 : Bios boot complete callback function flow	35

Figure 6 : POSC results callback function flow	37

Figure 7 : STL results callback function flow	39

Figure 8 : ODCC ss callback function flow	41

Figure 9 : Override config data callback function flow	43

Figure 10: Clock monitor error injection check function flow	45

Figure 11 : Clock monitor pre-mask status check function flow	47

Figure 12 : Clock monitor count done status function flow	49

Figure 13 : Clock monitor PPM range status check function flow	51

Figure 14: Clock monitor count error injection function flow	54

Figure 15: Clock monitor lock error injection function flow	57

Figure 16: Clock monitor dead clock error injection function flow	59

Figure 17: Root parity error injection check function flow	62

Figure 18: Boot FST frCPU check function flow	65

Figure 19: NVIC interrupt priority register check flow	67

Figure 20 : Async commands register function flow	69

Figure 21 : Async commands de-register function flow	71

Figure 22: Host monitoring application thread function flow	74

Figure 23: Host BIOS boot monitor overall flow diagram	76

Figure 24 : POSC results packet validate function flow	78

Figure 25 : POSC- get number of cores function flow	80

Figure 26: Host POSC results monitor flow diagram	83

Figure 27: RAVDM check function flow	84

Figure 28: RAVDM ping test function flow	86

Figure 29: Fetch ITD parameters function flow	88

Figure 30: PMC IPC test flow diagram	91

Figure 31: ODCC dummy snapshot compare flow diagram	93

Figure 32 : Override config data set	95

Figure 33 : Override config param function flow	97

Figure 34 : STL results packet validation function flow	99

Figure 35 : Uncore STL execution check function flow	100

Figure 36 : Core STL execution check function flow	102

Figure 37 : Syatem time set function flow	104

Figure 38: Host boot STLs monitor flow diagram	107

Figure 39: GP Timer expiry callback function flow	108

Figure 40 : Periodic Timer expiry callback function flow	110

Figure 41: RTOS timer test flow diagram	113

Figure 42 : WDT alarm mask and wdt start function	115

Figure 43 : WDT configure and start function flow	117

Figure 44: WDT test flow diagram	122

Figure 45 : GP timer difference calculation function flow	124

Figure 46: GP timer test flow diagram	126

Figure 47 : Periodic timer difference calculation function flow	128

Figure 48: Periodic timer test flow diagram	130

Figure 49: FMM scratchpad register check flow diagram	132

Figure 50: frCPU Multipler and Divider check flow	134

Figure 51: frCPU bus matrix check flow	138

Figure 52: frCPU MPU and NVIC register read function flow	140

Tables

Table 1 : Terminology	9

Table 2 : Reference Documents	10

Table 3 : Open & Follow up Needs	12

Table 4 : ISO26262 Fulfillment Matrix	12

Table 5 : IEC61508 Fulfillment Matrix	13

Table 6 : Boot FST PCIE message format for BIOS boot complete	16

Table 7 : Boot FST PCIE message format for POSC test results	17

Table 8 : Boot FST PCIE message format for startup STL results	18

Table 9 : Boot FST PCIE response message format	19

Table 10 : Boot FST PCIE message format for ODCC snapshot	19

Table 11 : Boot FST PCIE response message format for ODCC snapshot	20

Table 12 : Boot FST PCIE message format for override config data	20

Table 13 : Boot FST PCIE response message format for override config data	21

Table 14 : Boot FST PUINT request message format	22

Table 15 : Boot FST PUINT response message format	23

Table 16 : Boot FST PUINT ITD response data 0	23

Table 17 : Boot FST PUINT ITD response data 1	23

Table 18 : Boot FST PUINT ITD response data 2	24

Table 19 : Configuaration parameters	24

Table 20 : Boot FST initialization API	25

Table 21 : Boot FST stop API	26

Table 22 : Boot FST application thread function	28

Table 23 : Boot fst timer expiry callback function	31

Table 24 : Boot fst bios boot callback function	32

Table 25 : Boot fst POSC results callback function	34

Table 26 : Boot fst STL results callback function	36

Table 27 : Boot fst ODCC SS callback function	38

Table 28 : Boot fst override config param callback function	40

Table 29 : Clock monitor error injection check function	42

Table 30 : Clock monitor pre-mask status check function	45

Table 31 : Clock monitor count done status check function	46

Table 32 : Clock monitor PPM range status check function	48

Table 33 : Clock monitor count error injection function	50

Table 34 : Clock monitor lock error injection function	54

Table 35 : Clock monitor dead clock error injection function	56

Table 36 : Root parity error injection check function	59

Table 37 : ROM ECC error injection check function	62

Table 38 : Boot FST frCPU check function	63

Table 39 : frCPU NVIC interrupt priority register check function	65

Table 40 : Async commands register function	66

Table 41 : Async commands de-register function	68

Table 42 : Host monitoring application thread function	70

Table 43 : Host BIOS boot monitor function	73

Table 44 : POSC results packet validate function	75

Table 45 : POSC – get number of cores enabled function	77

Table 46 : POSC results respond function	79

Table 47 : POSC test results monitor function	80

Table 48 : RAVDM check function	82

Table 49 : RAVDM ping test function	83

Table 50 : Fetch ITD parameters function	86

Table 51 : PMC IPC check function	88

Table 52 : ODCC dummy snapshot compare	91

Table 53 : Override config param data set function	93

Table 54 Override config param handle function	95

Table 55 : STL results packet validation function	96

Table 56 : Uncore STL execution check function	98

Table 57 : Core STL execution check function	100

Table 58 : System time set function	101

Table 59 : STL results respond function	103

Table 60 : Host startup STL results monitor function	104

Table 61 : RTOS timer error handler function	109

Table 62 : Timer error injection check function	110

Table 63 : WDT alarm mask and wdt start function	113

Table 64 : WDT configure and start function	114

Table 65 : WDT difference error handle function	116

Table 66 : WDT error handler function	117

Table 67 : WDT NOK status error handle function	118

Table 68 : WDT check function	119

Table 69 : GP timer difference calculation function	122

Table 70 : GP timer check function	123

Table 71 : Periodic timer difference calculation function	126

Table 72 : Periodic timer check function	127

Table 73 : FMM scratchpad register check function	129

Table 74 : Multiplier and Divider check function	131

Table 75 : Bus Matrix check function	134

Table 76 : frCPU MPU and NVIC register read function	138

Table 77 : ThreadX errors	140

Table 78 : Global Data Description	141

Table 79 : Macros Data Description	142

Revision History

| Date | Revision | Author | Description |
| --- | --- | --- | --- |
| 05/21/18 | 0.30 | Amarnath C | Initial release for architectural and validation teams feedback |
| 06/27/18 | 0.31 | Amarnath C | Added flows for timer tests and host monitoring. |
| 11/02/18 | 0.50 | Amarnath C | Added more details |
| 12/14/18 | 0.55 | Amarnath C | Added more details |
| 01/28/19 | 0.60 | Amarnath C | - Updated PMC IPC test - Updated GP timer test |
| 02/08/19 | 0.8RC | Amarnath C | - Updated init, application and stop functions. - Added Clock monitor, Root parity and ROM ECC error injection tests. - Removed host monitor thread. - Added host monitor function, updated BIOS boot monitor, added POSC results monitor and updated startup STL results monitor. - Added periodic timer test and periodic timer expiry handle callback function. - Renamed RTOS timer test to Timer error injection test, updated details. - Updated details in WDT test. |
| 03/25/19 | 0.8RC | Amarnath C | Updated clock monitor test, root parity error injection test and RAVDM test. |
| 07/28/2020 | 1.0RC | Unmesh, Hanamshet | Updated flow diagrams and description for new implementation. |
| 01/07/2021 | 1.1RC | Amarnath C | Large functions are broken into small function, updated new functions added. |
| 25/02/2021 | 1.1.1RC | Kavya Rajesh Vembar | Updated clockmon error enjection test check flowchart |

# Introduction

## Purpose

This document describes the design details of platform boot FST. It is responsible for ensuring that the various Platform boot components are up.

SCI is a safety related sub-system present in platforms designed for industrial use such as Elkhart Lake based on which further discussions are carried out.

## Audience

The audience for this document is as follows:

- Safety auditors and reviewers for SCI

- Product design and development team for Elkhart Lake platform

- SCI firmware development and validation teams

- FUSA validation team for SCI

## Acronyms and Terminology

Table 1 : Terminology

| Term | Description |
| --- | --- |
| SCI | SCI stands for Sycamore Island, which is the 0.6 generation of Intel Safety Island. This conforms to IEC61508 standards and is meant primarily for industrial use-cases. |
| FST | Functional Safety Test |
| STL | Software Test Library |
| RAVDM | Register Access over Vendor Defined Message |
| FMM | Fault Management Module |

## Reference Documents

Table 2 : Reference Documents

| Document | Document No./Location |
| --- | --- |
| SCI Software Architecture Specification | https://sharepoint.amr.ith.intel.com/sites/FUSA_GlobalDomain/ISI/Sycamore%20Island%20ISI%2006/Engineering/Specifications/Sycamore%20Island%20SAS%20v0.8.pdf |
| JSL FuSa PM HAS for RAVDM formats | https://sharepoint.gar.ith.intel.com/sites/jasperlake/Shared%20Documents/Arch/HAS/PowerManagement/HAS/FuSa/JSL_FuSa_PM_HAS.html |
| fRCPU armcm3 SS A1.0 - SW and Application Safety Manual of fRCPU armcm3 product | https://sharepoint.amr.ith.intel.com/sites/FUSA_GlobalDomain/ISI/Safety%20Island%20WIP/Yogitech%20ISI/fRCPU_armcm3_SS_A1.0.pdf |
| Timer Service MDS | https://sharepoint.gar.ith.intel.com/sites/QSD/SitePages/Home.aspx?RootFolder=%2Fsites%2FQSD%2FShared%20Documents%2FISI%5Fsw%5Fcommon%5Fdata%2FTIMER%20MDS&FolderCTID=0x012000A4BA3FDD6692FE4DBE8956F8E2A8E438&View={7D387DC1-8317-4B5B-B1DB-E35DAE752468} |
| WDT, RTOS and GP timer MDS | https://sharepoint.gar.ith.intel.com/sites/QSD/SitePages/Home.aspx?RootFolder=%2Fsites%2FQSD%2FShared%20Documents%2FISI%5Fsw%5Fcommon%5Fdata%2FTIMER%20MDS%2FTIMER%20DRIVER%20MDS&FolderCTID=0x012000A4BA3FDD6692FE4DBE8956F8E2A8E438&View={7D387DC1-8317-4B5B-B1DB-E35DAE752468} |
| ISI diagnostic interface specifications | https://sharepoint.amr.ith.intel.com/sites/FUSA_GlobalDomain/ISI-FSXT/ISI%20Gen%2005%20Work%20Products/Forms/AllItems.aspx?RootFolder=%2fsites%2fFUSA_GlobalDomain%2fISI-FSXT%2fISI%20Gen%2005%20Work%20Products%2fExternal%2fISI%20Diagnostic%20Interface&FolderCTID=0x0120002BFEC25E27A6C749AC3DA4BFEDC77CE0 |
| SCI HAS and SCI components HAS SCI Clock Monitor MAS | https://sharepoint.amr.ith.intel.com/sites/FUSA_GlobalDomain/ISI/Safety%20Island%20WIP/Forms/AllItems.aspx?RootFolder=%2Fsites%2FFUSA%5FGlobalDomain%2FISI%2FSafety%20Island%20WIP%2FGen%200%2E6%20%28EHL%2DMCC%29%2FIP%20TR%20WG%2FSCI%5FHAS%2FLatest |

# Document Scope

## Prerequisite Documents

- Sycamore Island Software Architecture Specification documentation

## In Scope

This document is intended to capture the design details of boot FST for SCI.

## Out of Scope

## Known Gaps/Opens

Table 3 : Open & Follow up Needs

| Open and follow-up needs | Notes |
| --- | --- |
| - The flow shown for Error Handling is temporary. - [TBD] sharing the VID table to periodic FST. |  |

## ISO26262 and IEC61508 Fulfillment Matrix

This section identifies the content of this document which fulfills the identified safety requirements from the “In Scope” Section.

### ISO26262 Fulfillment Matrix

Table 4 : ISO26262 Fulfillment Matrix

| S. No | Technique/Measure | ASILD | Used | Section numbers within this document | Details |
| --- | --- | --- | --- | --- | --- |
| 1a | Natural language | ++ | Yes | All the figures from 1 to 15. | The complete document is written by combination of natural language and semi formal notations used for flow diagrams in the document. |
| 1c | Semi-formal notations | ++ | Yes | All the figures from 1 to 15. | Flow diagrams made in visio are using UML are semi formal methods. |

### IEC61508 Fulfillment Matrix

Table 5 : IEC61508 Fulfillment Matrix

| Table A4 - Part 3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| S. No |  | Technique/Measure | SIL 3 | Used | Section numbers within this document | Details |
| 1 |  | Semi-formal methods | HR | Yes | All the figures from 1 to 15. | Flow diagrams made in visio are using UML are semi-formal methods |
| 2 |  | Computer-aided design tools | R | Yes | All the figures from 1 to 15. | Visio is the computer aided design tool used for making flow diagrams. |
| 3 |  | Defensive programming | R | Yes | -- | Coding guidelines will impose restrictions to do defensive programming |
| 4 |  | Modular approach | HR | Yes | -- | The design of a module in broken down in smaller functional blocks(C functions, threads) responsible for performing a distinct functionality. |
| 5 |  | Design and coding standards | HR | Yes | -- | Coding guidelines address this measure |
| 6 |  | Structured programming | HR | Yes | -- | Coding guidelines address this measure |
| 7 |  | Use of trusted/verified software elements (if available) | HR | Yes | -- | Module uses APIs fromThreadX. ThreadX is compliant software developed as per IEC standards and qualified by TUV SUD or the same. |
| 8 |  | Forward traceability between the software safety requirements specification and software design | R | Yes | -- | Will be established in DNG |

# Global Design Decisions

Boot FST is the first FST to execute after SCI boot. The boot FST thread create and start is done by FSTM.

# Functional Description

## Overview

The Boot FST is launched by FSTM during boot and it is one of the first FSTs to run on SCI. This FST is responsible for carrying out:

- Proof test

It always checks the state of diagnostic virtual wires to CSE, if it is asserted (done in previous boot cycle by SCI), Boot FST will follow proof test flow and wait for results of proof test from CSE. If the wire is not asserted, then Boot FST follows its normal boot flow starting with next step below.

TODO: Update once all the details are available.

- Running various SCI Boot Safety Tests. These include:

  - Timer functional test(GP timer and periodic timer)

  - WDT functional test

  - Timer fault injection test

  - Clock monitor fault injection test

  - Root parity fault injection test

  - ROM ECC error injection test

  - FMM Scratch pad test

  - fRCPU Boot test

- Host boot monitoring

  - Boot complete from BIOS

    - Host BIOS on its boot completion sends a message to SCI via PCIe MB. On receiving boot complete message SCI will send the acknowledgement with success response code to host. If host does not receive acknowledgement in time, host will resend the boot complete message to SCI. This message is used as trigger point to fetch the ITD parametrers from PUINT.

  - POSC test results:

    - Host sends the Pre-OS checker test results message to SCI. This message informs SCI about the number of cores from whom STL results to be expected. SCI will send the acknowledgement with success response code to host. If host does not receive acknowledgement in time, host will resend the POSC test results to SCI.

  - RAVDM test

    - Send ping command to check communication between SCI and PUNIT. Receive the ping command response from PUNIT.

    - Also, send command to get ITD parameter and calculate the voltage values corresponding to temperature.

  - PMC IPC test

    - To test the PMC IPC communication, send the get platform transition information Query Command to PMC and receive the response.

  - ODCC test

    - Compare ODCC snapshots coming from host test application if it comes while boot FST is executing. Host can use this to test ODCC software safety mechanism. ODCC comparison should detect mismatch in incoming snapshots if incoming snapshots do not match.

TODO: Update once all the details are available.

  - Startup STL results

    - On host boot complete, it sends the startup STL results to SCI from cores (as informed by POSC test results). SCI sends PST and DTI configurations to host safety application running startup STL in response to STL results.

If Boot FST finds out any error in any of these Host Boot Tests and SCI Boot STL, it will signal the FSTM about it. FSTM will assert NOK, invoke required error handling and not get SCI out of the Safe State.

Refer ISI diagnostic interface specifications for host message and response format.

Refer JSL FuSa PM HAS for RAVDM formats.

## Type Definitions

### Local type definitions

Table 6 : Boot FST PCIE message format for BIOS boot complete

| Syntax: | struct boot_fst_bios_boot_msg |
| --- | --- |
| Type: | struct |  |
| File: | boot_fst.c |  |
| Range: |  |  |
| Description: | PCIe MB message format for BIOS boot complete. |  |
| Members: | uint32_t version : 4U; | Version of specifications. |
|  | uint32_t reserved_1 : 4U; | Reserved. |
|  | uint32_t reserved_2 : 6U; | Reserved. |
|  | uint32_t flags : 2U; | Flag for response bit and retry bit. |
|  | uint32_t length : 8U; | Message length in DWORDs. |
|  | uint32_t reserved_3 : 8U; | Reserved. |
|  | uint32_t seq_num : 16U; | Sequence number. |
|  | uint32_t wl_address : 8U; | Workload address. |
|  | uint32_t isi_address : 8U; | ISI address. |
|  | uint32_t crc : 16U; | CRC value. |
|  | uint32_t command : 16U; | BIOS boot complete command. |

Table 7 : Boot FST PCIE message format for POSC test results

| Syntax: | struct boot_fst_posc_res_msg |
| --- | --- |
| Type: | struct |  |
| File: | boot_fst.c |  |
| Range: |  |  |
| Description: | PCIe MB message format for POSC test results. |  |
| Members: | uint32_t version : 4U; | Version of specifications. |
|  | uint32_t reserved_1 : 4U; | Reserved. |
|  | uint32_t reserved_2 : 6U; | Reserved. |
|  | uint32_t flags : 2U; | Flag for response bit and retry bit. |
|  | uint32_t length : 8U; | Message length in DWORDs. |
|  | uint32_t reserved_3 : 8U; | Reserved. |
|  | uint32_t seq_num : 16U; | Sequence number. |
|  | uint32_t wl_address : 8U; | Workload address. |
|  | uint32_t isi_address : 8U; | ISI address. |
|  | uint32_t crc : 16U; | CRC value. |
|  | uint32_t command : 16U; | POSC test results command. |
|  | uint32_t posc_res : 16U; | POSC result. |
|  | uint32_t posc_maj : 8U; | POSC major version. |
|  | uint32_t posc_min : 8U; | POSC minor version. |
|  | uint32_t core_mask_1; | Core STL executed or not executed. |
|  | uint32_t core_mask_2; | Core STL executed or not executed. |
|  | uint32_t avx_data_valid; |  |
|  | uint32_t sse_freq; |  |
|  | uint32_t avx_freq; |  |
|  | uint32_t avx2_freq; |  |
|  | uint32_t core_mask1_sse_0_15; | Core mask |
|  | uint32_t core_mask2_sse_16_31; | Core mask |
|  | uint32_t core_mask1_avx_0_15; | Core mask |
|  | uint32_t core_mask2_avx_16_31; | Core mask |
|  | uint32_t core_mask1_avx2_0_15; | Core mask |
|  | uint32_t core_mask2_avx2_16_31; | Core mask |
|  | uint32_t core_mask1_red_green1_0_15; | Core mask |
|  | uint32_t core_mask2_red_green2_16_31; | Core mask |

Table 8 : Boot FST PCIE message format for startup STL results

| Syntax: | struct boot_fst_stl_res_msg |
| --- | --- |
| Type: | struct |  |
| File: | boot_fst.c |  |
| Range: |  |  |
| Description: | PCIe MB message format for startup STL results. |  |
| Members: | uint32_t version : 4U; | Version of specifications. |
|  | uint32_t reserved_1 : 4U; | Reserved. |
|  | uint32_t reserved_2 : 6U; | Reserved. |
|  | uint32_t flags : 2U; | Flag for response bit and retry bit. |
|  | uint32_t length : 8U; | Message length in DWORDs. |
|  | uint32_t reserved_3 : 8U; | Reserved. |
|  | uint32_t seq_num : 16U; | Sequence number. |
|  | uint32_t wl_address : 8U; | Workload address. |
|  | uint32_t isi_address : 8U; | ISI address. |
|  | uint32_t crc : 16U; | CRC value. |
|  | uint32_t command : 16U; | POSC test results command. |
|  | uint32_t sys_time_validity : 8U; | System time valid or not. |
|  | uint32_t stl_res_validity : 8U; | STL results valid or not. |
|  | uint32_t stl_res : 8U; | STL result. |
|  | uint32_t uncore_mask : 8U; | Uncore mask is enable/disable. |
|  | uint32_t core_mask_1 | Core STL executed or not executed. |
|  | uint32_t core_mask_2 | Core STL executed or not executed. |
|  | uint32_t sys_time; | System time in POSIX format. |

Table 9 : Boot FST PCIE response message format

| Syntax: | struct boot_fst_pcie_res_s |
| --- | --- |
| Type: | Struct |  |
| File: | boot_fst.c |  |
| Range: |  |  |
| Description: | PCIe MB response message format. |  |
| Members: | uint32_t version : 4U; | Version of specifications. |
|  | uint32_t reserved_1 : 4U; | Reserved. |
|  | uint32_t reserved_2 : 6U; | Reserved. |
|  | uint32_t flags : 2U; | Flag for response bit and retry bit. |
|  | uint32_t length : 8U; | Message length in DWORDs. |
|  | uint32_t reserved_3 : 8U; | Reserved. |
|  | uint32_t seq_num : 16U; | Sequence number. |
|  | uint32_t wl_address : 8U; | Workload address. |
|  | uint32_t isi_address : 8U; | ISI address. |
|  | uint32_t crc : 16U; | CRC value. |
|  | uint32_t command : 16U; | command. |
|  | uint32_t res_code : 16U; | Response code. |
|  | uint32_t reserved_4 : 16U; | Reserved |
|  | uint32_t pst : 16U; | PST value. |
|  | uint32_t dti : 16U; | DTI value. |

Table 10 : Boot FST PCIE message format for ODCC snapshot

| Syntax: | struct boot_fst_odcc_ss_msg |
| --- | --- |
| Type: | struct |  |
| File: | boot_fst.c |  |
| Range: |  |  |
| Description: | PCIe MB message format for ODCC snapshot. |  |
|  | struct tpf_host_mcu_packet_header packet_header | ODCC packet header. |
|  | uint32_t channel_id : 8U | Channel Id |
|  | uint32_t workload_id : 8U | Workload Id |
|  | uint32_t snapshot_id : 8U | Snapshot Id |
|  | uint32_t reserved_3 : 8U | Reserved |
|  | uint32_t reserved_4 | Reserved |
|  | uint32_t signature | ODCC Signature |

Table 11 : Boot FST PCIE response message format for ODCC snapshot

| Syntax: | struct boot_fst_odcc_ss_res |
| --- | --- |
| Type: | struct |  |
| File: | boot_fst.c |  |
| Range: |  |  |
| Description: | PCIe MB message format for ODCC snapshot response. |  |
|  | struct tpf_host_mcu_packet_header packet_header | ODCC packet header. |
|  | uint32_t channel_id : 8 | Channel Id |
|  | uint32_t workload_id : 8 | Workload Id |
|  | uint32_t snapshot_id : 8 | Snapshot Id |
|  | uint32_t response : 8 | Response code |

Table 12 : Boot FST PCIE message format for override config data

| Syntax: | struct boot_fst_write_config_data_cmd_s |
| --- | --- |
| Type: | struct |  |
| File: | boot_fst.c |  |
| Range: |  |  |
| Description: | PCIe MB message format for override config data. |  |
|  | struct tpf_host_mcu_packet_header hdr | Override config data packet header. |
|  | uint32_t pst | Payload data received. |
|  | uint32_t prochot : 8 |  |
|  | uint32_t log_level : 8 |  |
|  | uint32_t mission_mode : 8 |  |
|  | uint32_t rsvd_1 : 8 |  |
|  | uint32_t all_sci_warn_thresh : 8 |  |
|  | uint32_t all_sci_warn_cnt_ref_interval : 16 |  |
|  | uint32_t rsvd_2 : 8 |  |
|  | uint32_t pch_err1_thresh : 8 |  |
|  | uint32_t pch_err1_cnt_ref_interval : 16 |  |
|  | uint32_t rsvd_3 : 8 |  |
|  | uint32_t pch_err0_thresh : 8 |  |
|  | uint32_t pch_err0_cnt_ref_interval : 16 |  |
|  | uint32_t rsvd_4 : 8 |  |
|  | uint32_t ibecc_dram_err_thresh : 8 |  |
|  | uint32_t ibecc_dram_err_cnt_ref_interval : 16 |  |
|  | uint32_t rsvd_5 : 8 |  |
|  | uint32_t cmci_err_thresh : 8 |  |
|  | uint32_t cmci_err_cnt_ref_interval : 16 |  |
|  | uint32_t rsvd_6 : 8 |  |

Table 13 : Boot FST PCIE response message format for override config data

| Syntax: | struct boot_fst_write_config_data_resp_s |
| --- | --- |
| Type: | struct |  |
| File: | boot_fst.c |  |
| Range: |  |  |
| Description: | PCIe MB response message format for override config data. |  |
|  | struct tpf_host_mcu_packet_header hdr | Override config data packet header. |
|  | struct { uint32_t resp_code : 16 uint32_t pst_invalid : 1 uint32_t prochot_out_of_range : 1 uint32_t log_level_invalid : 1 uint32_t mission_mode_invalid : 1 uint32_t all_sci_warn_thresh_out_of_range : 1 uint32_t all_sci_warn_ref_int_out_of_range : 1 uint32_t pch_err1_thresh_out_of_range : 1 uint32_t pch_err1_cnt_ref_int_out_of_range : 1 uint32_t pch_err0_thresh_out_of_range : 1 uint32_t pch_err0_cnt_ref_int_out_of_range : 1 uint32_t ibecc_dram_err_thresh_out_of_range : 1 uint32_t ibecc_dram_err_cnt_ref_int_out_of_range : 1 uint32_t cmci_err_thresh_out_of_range : 1 uint32_t cmci_err_cnt_ref_int_out_of_range : 1 uint32_t rsvd : 2 } payload; | Payload data. |

Table 14 : Boot FST PUINT request message format

| Syntax: | struct boot_fst_punit_request |  |
| --- | --- | --- |
| Type: | struct |  |
| File: | boot_fst.c |  |
| Range: |  |  |
| Description: | PUNIT request message format for PING test and fetching ITD parameters. |  |
| Members: | uint32_t cmd_val : 8 | Command value. |
|  | uint32_t seqno : 4 | Sequence number. |
|  | uint32_t reserved1 : 4 | Reserved area. |
|  | uint32_t crc : 8 | CRC value. |
|  | uint32_t reserved2 : 4 | Reserved area. |
|  | uint32_t reserved3 : 4 | Reserved area. |
|  | uint32_t data | Data. |

Table 15 : Boot FST PUINT response message format

| Syntax: | struct boot_fst_punit_response |  |
| --- | --- | --- |
| Type: | struct |  |
| File: | boot_fst.c |  |
| Range: |  |  |
| Description: | Response message from PUINT. |  |
| Members: | uint32_t cmd_val : 8 | Command value. |
|  | uint32_t seqno : 4 | Sequence number. |
|  | uint32_t res_code : 4 | Response code. |
|  | uint32_t num_dwords : 8 | Number of DWORDS in message payload. |
|  | uint32_t reserved1 : 4 | Reserved area. |
|  | uint32_t reserved2 : 4 | Reserved area. |
|  | uint32_t data[BOOT_FST_PUNIT_RES_DATA_SIZE = 16] | Data. |
|  | uint32_t crc | CRC value. |

Table 16 : Boot FST PUINT ITD response data 0

| Syntax: | struct boot_fst_itd_res_data_0 |  |
| --- | --- | --- |
| Type: | struct |  |
| File: | boot_fst.c |  |
| Range: |  |  |
| Description: | ITD response data 0. |  |
| Members: | uint32_t itd_cutoff_tj_val : 7U | ITD_CUTOFF_TJ. |
|  | uint32_t resvd1 : 9U | Reserved area. |
|  | uint32_t gauranteed_vid : 16U | Domain's Guaranteed VID. |

Table 17 : Boot FST PUINT ITD response data 1

| Syntax: | struct boot_fst_itd_res_data_1 |  |
| --- | --- | --- |
| Type: | struct |  |
| File: | boot_fst.c |  |
| Range: |  |  |
| Description: | ITD response data 1. |  |
| Members: | uint32_t itd_cutoff_v_val : 9U | ITD_CUTOFF_V. |
|  | uint32_t resvd1 : 7U | Reserved area. |
|  | uint32_t itd_cutoff_v2_val : 9U | ITD_CUTOFF_V2. |
|  | uint32_t resvd2 : 7U | Reserved area. |

Table 18 : Boot FST PUINT ITD response data 2

| Syntax: | struct boot_fst_itd_res_data_2 |  |
| --- | --- | --- |
| Type: | struct |  |
| File: | boot_fst.c |  |
| Range: |  |  |
| Description: | ITD response data 2. |  |
| Members: | uint32_t slope : 8U | SLOPE. |
|  | uint32_t resvd1 : 8U | Reserved area. |
|  | uint32_t slope2 : 8U | SLOPE2. |
|  | uint32_t resvd2 : 8U | Reserved area. |

## Configuration Parameters

Table 19 : Configuaration parameters

| Syntax: | struct boot_fst_config_s |
| --- | --- |
| Type: | struct |  |
| File: | Platform_config_defs.h (TBD: move it to boot_fst.h) |  |
| Range: |  |  |
| Description: | Boot FST configuration parameters. TODO: Finalize members. |  |
|  | uint32_t bios_boot_time; | BIOS boot time. |
|  | uint32_t posc_results_time; | Getting POSC results time. |
|  | uint32_t stl_results_time; | Getting startup STL results time. |

## API Functions

### Boot FST initialization

Table 20 : Boot FST initialization API

| Service name: | boot_fst_init |  |  |
| --- | --- | --- | --- |
| Syntax: | uint32_t boot_fst_init ( void *config_param ); |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | *config_param | void | The void * pointer points to the config_param corresponding to this FST, as set by the FSTM. The FSTM itself gets the Config Parameter from the SRAM, placed by SCI ROM. |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | Boot FST init successful. |
|  | STATUS_THREADX(M_FST_BOOT, E_IN_INIT, tx_status) |  | On event flag creation/byte allocation/queue creation failure. |
|  | STATUS_FIRMWARE(M_FST_BOOT, E_IN_INIT, E_INVALID_ARGUMENT); |  | On host monitor timer value equal to zero. |
|  | STATUS_FIRMWARE(M_FST_BOOT, E_IN_INIT, E_INVALID_PTR) |  | On NULL input parameter. |
| Description: | This API performs the initialization of various globals and config parameters corresponding to this FST as sent in the void *config_param from the FSTM. |  |  |
| Preconditions: | Boot FST must be created. |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful initialization, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Validate the input parameter. - Validate the host monitor timer timeout value in input configurations. Store the input configurations. - Create event flag for monitoring the host boot. - Allocate memory to boot FST queue and create the queue. - If any error in above steps, report to upper layer. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 1: Boot FST initialization flow

### Boot FST stop

Table 21 : Boot FST stop API

| Service name: | boot_fst_stop |  |  |
| --- | --- | --- | --- |
| Syntax: | uint32_t boot_fst_stop ( void ); |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | Boot FST stop successful. |
|  | STATUS_FIRMWARE |  | On firmware failure. |
|  | STATUS_THREADX(M_FST_BOOT, E_IN_STOP, tx_status) |  | On BIOS stage2 timer deactivate or delete failed. |
| Description: | This API terminates and deletes every resource created by boot FST and related memory allocated. Also gets the fst app and host monitoring app threads out of any blocked state, so that the boot FST thread can be terminated and deleted by FSTM. |  |  |
| Preconditions: | Boot FST must be created and initialized. |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful stop, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API deletes the host monitor event flag, boot FST queue and releases the memory allocated for queue. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 2: Boot FST stop flow

### Boot FST application

Table 22 : Boot FST application thread function

| Service name: | boot_fst_app |  |  |
| --- | --- | --- | --- |
| Syntax: | void boot_fst_app ( uint32_t thread_input ); |  |  |
| Sync/Async: | Asynchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | thread_input | uint32_t | Zeror. |
| Parameters (out): | void |  |  |
| Return value: | void |  |  |
| Description: | This API implements the actual Boot FST functionality. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: |  |  |  |
| Implementation comments | This API performs the following operations. - Calls fst_comman_rtos_timer_check() to test the RTOS timer. - Calls fst_common_gp_timer_check() to test the GP timer. - Calls fst_common_prd_timer_check() to test the periodic timer. - Call fst_common_wdt_check() to test the WDT. - Call fst_common_fmm_sp_reg_check() to test FMM scratchpad register. - Call boot_fst_frcpu_check() to run frCPU tests. - Calls boot_fst_clk_mon_ei_check() to test clock monitor error injection. - Calls boot_fst_root_parity_ei_check() to test root parity error injection. - Mask WDT timeout FMM alarm. - Calls boot_fst_host_monitor() to monitor host tests. - Start WDT and unmask WDT timeout FMM alarm. - Report status to FSTM program flow monitoring. - Calls fw_internal_error_handler API if status is failure. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 590398, 850153 |  |  |

Figure 3: Boot FST app flow

## Interrupt Handler

NA

## Local functions

### Boot FST timer expiry callback function

Table 23 : Boot fst timer expiry callback function

| Service name: | boot_fst_timer_expiry_fn |  |  |
| --- | --- | --- | --- |
| Syntax: | static void boot_fst_timer_expiry_fn( timer_id_e id ) |  |  |
| Sync/Async: | Asynchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | id | timer_id_e | Timer id. |
| Parameters (out): | void |  |  |
| Return value: | void |  |  |
| Description: | This API is called on boot FST timer expiry. |  |  |
| Preconditions: | This API must be registered while creating the boot fst timers. |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: |  |  |  |
| Implementation comments | This API validates the timer expired, updates the diagnostic data and calls errmgt_firmware_internal_error() API to assert NOK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 4: Boot fst Timer expiry callback function flow

### Boot FST bios boot callback function

Table 24 : Boot fst bios boot callback function

| Service name: | Boot FST bios boot callback function |  |  |
| --- | --- | --- | --- |
| Syntax: | static void boot_fst_bios_boot_cb ( void *data, uint32_t len) ) |  |  |
| Sync/Async: | Asynchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | *data | void | Pointer to input data. |
|  | len | Uint32_t | Length of input data. |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful data receiving. |
|  | STATUS_THREADX(M_FST_BOOT, E_IN_CALLBACK1, tx_status) |  | On queue send failure. |
|  | STATUS_THREADX(M_FST_BOOT, E_IN_CALLBACK2, tx_status) |  | On event flag set failure. |
| Description: | This API is called on receiving the async message from host. |  |  |
| Preconditions: | This API must be registered with FSTM async interface layer. |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful receiving data, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Calls tx_event_flags_set() to notify bios boot message reception. - Calls tx_queue_send() to send the data to boot fst thread. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 5 : Bios boot complete callback function flow

### Boot FST POSC results callback function

Table 25 : Boot fst POSC results callback function

| Service name: | Boot FST POSC results callback function |  |  |
| --- | --- | --- | --- |
| Syntax: | static void boot_fst_posc_results_cb ( void *data, uint32_t len) ) |  |  |
| Sync/Async: | Asynchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | *data | void | Pointer to input data. |
|  | len | Uint32_t | Length of input data. |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful data receiving. |
|  | STATUS_THREADX(M_FST_BOOT, E_IN_CALLBACK1, tx_status) |  | On queue send failure. |
|  | STATUS_THREADX(M_FST_BOOT, E_IN_CALLBACK2, tx_status) |  | On event flag set failure. |
| Description: | This API is called on receiving the async message from host. |  |  |
| Preconditions: | This API must be registered with FSTM async interface layer. |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful receiving data, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Calls tx_event_flags_set() to notify POSC results message reception. - Calls tx_queue_send() to send the data to boot fst thread. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 6 : POSC results callback function flow

### Boot FST STL results callback function

Table 26 : Boot fst STL results callback function

| Service name: | Boot FST STL results callback function |  |  |
| --- | --- | --- | --- |
| Syntax: | static void boot_fst_stl_results_cb ( void *data, uint32_t len) ) |  |  |
| Sync/Async: | Asynchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | *data | void | Pointer to input data. |
|  | len | Uint32_t | Length of input data. |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful data receiving. |
|  | STATUS_THREADX(M_FST_BOOT, E_IN_CALLBACK1, tx_status) |  | On queue send failure. |
|  | STATUS_THREADX(M_FST_BOOT, E_IN_CALLBACK2, tx_status) |  | On event flag set failure. |
| Description: | This API is called on receiving the async message from host. |  |  |
| Preconditions: | This API must be registered with FSTM async interface layer. |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful receiving data, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Calls tx_event_flags_set() to notify STL results message reception. - Calls tx_queue_send() to send the data to boot fst thread. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 7 : STL results callback function flow

### Boot FST ODCC SS callback function

Table 27 : Boot fst ODCC SS callback function

| Service name: | Boot FST ODCC Snap Shot callback function |  |  |
| --- | --- | --- | --- |
| Syntax: | static void boot_fst_odcc_ss_cb ( void *data, uint32_t len) ) |  |  |
| Sync/Async: | Asynchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | *data | void | Pointer to input data. |
|  | len | Uint32_t | Length of input data. |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful data receiving. |
|  | STATUS_THREADX(M_FST_BOOT, E_IN_CALLBACK1, tx_status) |  | On queue send failure. |
|  | STATUS_THREADX(M_FST_BOOT, E_IN_CALLBACK2, tx_status) |  | On event flag set failure. |
| Description: | This API is called on receiving the async message from host. |  |  |
| Preconditions: | This API must be registered with FSTM async interface layer. |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful receiving data, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Calls tx_event_flags_set() to notify ODCC SS message reception. - Calls tx_queue_send() to send the data to boot fst thread. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 8 : ODCC ss callback function flow

### Boot FST override config param callback function

Table 28 : Boot fst override config param callback function

| Service name: | Boot FST override config param callback function |  |  |
| --- | --- | --- | --- |
| Syntax: | static void boot_fst_override_config_param_cb ( void *data, uint32_t len) ) |  |  |
| Sync/Async: | Asynchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | *data | void | Pointer to input data. |
|  | len | Uint32_t | Length of input data. |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful data receiving. |
|  | STATUS_THREADX(M_FST_BOOT, E_IN_CALLBACK1, tx_status) |  | On queue send failure. |
|  | STATUS_THREADX(M_FST_BOOT, E_IN_CALLBACK2, tx_status) |  | On event flag set failure. |
| Description: | This API is called on receiving the async message from host. |  |  |
| Preconditions: | This API must be registered with FSTM async interface layer. |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful receiving data, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Calls tx_event_flags_set() to notify override config param message reception. - Calls tx_queue_send() to send the data to boot fst thread. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 9 : Override config data callback function flow

### Clock monitor Error Injection(EI) test

Table 29 : Clock monitor error injection check function

| Service name: | boot_fst_clk_mon_ei_check |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_clk_mon_ei_check (void); |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | Clock monitor error injection check successful. |
|  | STATUS_FIRMWARE (M_FST_BOOT, E_IN_CLK_MONITOR_ERR_INJECTION, E_INVALID_CLK_MONITOR_ID) |  | Clock monitor error injection check failed. |
| Description: | This API performs the Clock monitor error injection test. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful Clock monitor error injection check, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - In a for loop of NUM_CLK_MONITOR (defined in local control driver) perform the below steps. - Calls lc_osdl_clkmon_status_set() to enable clock monitor. - Calls the boot_fst_count_ei(clock monitor id, sample window, target count and error count) to test the count error injection. - Calls the boot_fst_lock_ei (clock monitor id, sample window, target count) to test the lock error injection. - Calls the boot_fst_dead_clock_ei (clock monitor id) to test the dead clock error injection. - Call the lc_osdl_clkmon_set_val(CLK_MONITOR_2, CLKMON_SAMPLE_WINDOW, 192302U) and lc_osdl_clkmon_set_val( CLK_MONITOR_2, CLKMON_TARGET_COUNT, 1001573U) to set the sample window and target count - Call the lc_osdl_clkmon_set_val(CLK_MONITOR_3, CLKMON_SAMPLE_WINDOW, 1001573U) and lc_osdl_clkmon_set_val( CLK_MONITOR_3, CLKMON_TARGET_COUNT, 192302U) to set the sample window and target count - Calls lc_osdl_clkmon_status_set() to disable clock monitor - If any error in above steps, return error . |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 850142 |  |  |

Figure 10: Clock monitor error injection check function flow

### Clock monitor pre-mask status check

Table 30 : Clock monitor pre-mask status check function

| Service name: | boot_fst_cm_pre_mask_status_check |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_cm_pre_mask_status_check ( enum clk_monitor_id_e clk_mon_id, uint32_t sample_window, uint32_t target_count, uint32_t error_count, uint8_t ei_type ) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | Clock monitor pre-mask status successful. |
|  | Return value of errmgt_firmware_internal_error() API. |  |  |
| Description: | This API gets pre-mask alarm status for clock mon id.. Note: The flow is covered as part of calling function flow diagram. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful status check, else it will error handling API to assert NOK. |  |  |
| Implementation comments | This API performs the following operations. - Calls lc_osdl_clkmon_status_get() using parameters clk_mon_id, CLKMON_PRE_MASK_ALARM, &comp_status to get the pre mask alarm status. - If alarm is not set, test failed, update the diagnostic data and calls errmgt_firmware_internal_error() API to assert NOK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 11 : Clock monitor pre-mask status check function flow

### Clock monitor count done status check

Table 31 : Clock monitor count done status check function

| Service name: | boot_fst_cm_count_done_status_check |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_cm_count_done_status_check ( enum clk_monitor_id_e clk_mon_id, uint32_t sample_window, uint32_t target_count, uint32_t error_count ) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | Clock monitor count done status get successful. |
|  | Return value of errmgt_firmware_internal_error() API. |  |  |
| Description: | This API waits for count done status and checks count done status. Note: The flow is covered as part of calling function flow diagram. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful status check, else it will error handling API to assert NOK. |  |  |
| Implementation comments | This API performs the following operations. - Calls tx_thread_sleep to wait for count done status to set. - Calls lc_osdl_clkmon_status_get() using parameters clk_mon_id, CLKMON_COUNT_DONE_STATUS, &comp_status to get the count done status. - If status is not set, test failed, update the diagnostic data and calls errmgt_firmware_internal_error() API to assert NOK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 12 : Clock monitor count done status function flow

### Clock monitor PPM range status check

Table 32 : Clock monitor PPM range status check function

| Service name: | boot_fst_cm_ppm_range_status_check |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_cm_ppm_range_status_check ( enum clk_monitor_id_e clk_mon_id, uint32_t sample_window, uint32_t target_count, uint32_t error_count ) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | Clock monitor ppm range status get successful. |
|  | Return value of errmgt_firmware_internal_error() API. |  |  |
| Description: | This API gets PPM range status if not set calls error hanlding API. Note: The flow is covered as part of calling function flow diagram. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful status check, else it will error handling API to assert NOK. |  |  |
| Implementation comments | This API performs the following operations. - Calls lc_osdl_clkmon_status_get() using parameters clk_mon_id, CLKMON_PPM_IN_RANGE_STATUS, &comp_status to get PPM IN range status. - If status is not set, test failed, update the diagnostic data and calls errmgt_firmware_internal_error() API to assert NOK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 13 : Clock monitor PPM range status check function flow

### Clock monitor count error injection test

Table 33 : Clock monitor count error injection function

| Service name: | boot_fst_count_ei |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_count_ei( enum clk_monitor_id_e clk_mon_id, uint32_t sample_window, uint32_t target_count, uint32_t error_count ) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | Clock monitor count error injection check successful. |
|  | STATUS_FIRMWARE ( M_FST_BOOT, E_IN_COUNT_ERR_INJECTION, E_STATUS_BITS_MISMATCH) |  | Clock monitor count error injection check failed. |
| Description: | This API performs the Clock monitor count error injection test. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful Clock monitor count error injection check, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Calls the lc_osdl_clkmon_set_val() using parameters clk_mon_id, CLKMON_SAMPLE_WINDOW and sample_window to program sample window. - Calls the lc_osdl_clkmon_set_val() using parameters clk_mon_id, CLKMON_TARGET_COUNT and target_count to program target count. - Calls lc_osdl_clkmon_set_val() parameters clk_mon_id, CLKMON_ERR_COUNT, error_count to program error count. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_CTRL_IN_CNT_ERR_INJ_EN, True to enable the count error injection. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_CTRL_IN_STOP_REPEATand False to disable repeat mode. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_MASK_NOK_ERR_INJ andTrue to enable NOK mask. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_CTRL_IN_ENABLE and True to enable clock monitor. - Calls boot_fst_cm_count_done_status_check() API to check count done status. - Calls boot_fst_cm_ppm_range_status_check() API to check ppm range status. - Calls boot_fst_cm_pre_mask_status_check() API to check pre-mask status. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_CTRL_IN_ENABLE, False to disable clock monitor. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_CTRL_IN_CNT_ERR_INJ_EN, False to disable count error injection. - Calls lc_osdl_clkmon_status_set() using clk_mon_id, CLKMON_MASK_NOK_ERR_INJ, False to disable NOK mask. - If any error in above steps, return error . |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 14: Clock monitor count error injection function flow

### Clock monitor lock error injection test

Table 34 : Clock monitor lock error injection function

| Service name: | boot_fst_lock_ei |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_lock_ei ( enum clk_monitor_id_e clk_mon_id, uint32_t sample_window, uint32_t target_count ) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | Clock monitor lock error injection check successful. |
|  | STATUS_FIRMWARE |  | Clock monitor lock error injection check failed. |
| Description: | This API performs the Clock monitor lock error injection test. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful clock monitor lock error injection check, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Calls the lc_osdl_clkmon_set_val() using parameters clk_mon_id, CLKMON_SAMPLE_WINDOW, sample_window to program sample window. - Calls the lc_osdl_clkmon_set_val() using parameters clk_mon_id, CLKMON_TARGET_COUNT, target_count to program target count. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_MASK_NOK_ERR_INJ, True to enable NOK mask. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_CTRL_IN_ENABLE, True to enable clock monitor. - Wait for minimum of 10 period clocks. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_CTRL_IN_LOCK_ERR_INJ_EN, True to enable the lock error injection. - Calls lc_osdl_clkmon_status_get() using parameters clk_mon_id, CLKMON_LOCK_ERROR_STATUS, &comp_status to get the lock error status. If status is equal to 0x2 then test pass else failed. - Calls boot_fst_cm_pre_mask_status_check() API to check pre-mask status.s - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_CTRL_IN_ENABLE, False to disable clock monitor. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_CTRL_IN_LOCK_ERR_INJ_EN, False to disable lock error injection. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_MASK_NOK_ERR_INJ, False to disable NOK mask. - If any error in above steps, return error . |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 15: Clock monitor lock error injection function flow

### Clock monitor dead clock error injection test

Table 35 : Clock monitor dead clock error injection function

| Service name: | boot_fst_dead_clock_ei |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_dead_clock_ei ( enum clk_monitor_id_e clk_mon_id, ) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | Clock monitor dead clock error injection check successful. |
|  | STATUS_FIRMWARE |  | Clock monitor dead clock error injection check failed. |
| Description: | This API performs the Clock monitor dead clock error injection test. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful clock monitor dead clock error injection check, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_DCD_ERROR_INJ_EN, True to enable the dead clock error injection. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_MASK_NOK_ERR_INJ, True) to enable NOK mask. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_CTRL_IN_ENABLE, True to enable clock monitor. - Wait for timeout period. - Calls lc_osdl_clkmon_status_get() using parameters clk_mon_id, CLKMON_DCD_ERROR_STATUS, &comp_status to get the dead clock error status. If error status is 0x2 then test is pass else fail. - Calls boot_fst_cm_pre_mask_status_check() API to check pre-mask status. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_CTRL_IN_ENABLE, False to disable clock monitor. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_DCD_ERROR_INJ_EN, False to disable dead clock error injection. - Calls lc_osdl_clkmon_status_set() using parameters clk_mon_id, CLKMON_MASK_NOK_ERR_INJ, False) to disable NOK mask. - If any error in above steps, return error . |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 16: Clock monitor dead clock error injection function flow

### Root parity Error Injection(EI) test

Table 36 : Root parity error injection check function

| Service name: | boot_fst_root_parity_ei_check |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_root_parity_ei_check (void); |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | Root parity error injection check successful. |
|  | STATUS_FIRMWARE |  | Root parity error injection check failed. |
| Description: | This API performs the root parity error injection test for DMA, RTOS, WDT, PSS, FMM, GPIO, Global control and local control sub modules. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful root parity error injection check, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Calls fmm_parity_alarm_mask_unmask() to mask root parity alarm. - Calls fmm_parity_alarm_inject() to enable root parity error injection. - Calls fmm_parity_alarm_premask_status_get() to get the pre-mask alarm status bit. If pre-mask alarm is generated, test pass, else test failed. If test failed update the status with failure value. - Calls fmm_parity_alarm_inject_disable() to disable root parity error injection. - Calls fmm_parity_alarm_clear() to clear the pre-mask alarm. - Calls fmm_parity_alarm_mask_unmask() to unmask root parity alarm. - If any error in above steps, return error. - Repeate the above steps for all sub-modules in a for loop. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 850142 |  |  |

Figure 17: Root parity error injection check function flow

### ROM ECC error injection test

Table 37 : ROM ECC error injection check function

| Service name: | boot_fst_rom_ecc_ei_check |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_rom_ecc_ei_check ( void); |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | frCPU check successful. |
|  | STATUS_FIRMWARE |  | frCPU check failed. |
| Description: | This API performs the ROM ECC error injection test. Note: This requirement should come from CCB. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful frCPU check, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | The proposal is to perform following operations. - Mask FMM’s ROM_ECC_ucerr_alarm. - Program DMA for ROM data access to the single/multi bit error location. - Wait DMA interrupt for completed transfer then check/clear AXI_Status_reg in DMA (how is that cleared?). - Check FMM’s ROM_ECC_ucerr_alarm pre_status to make sure it is set (or check Alarm Status Register in MEMSS), NOK if not set. - Clear Alarm Status Register and Info Status Register in MEMSS. - Unmask FMM’s ROM_ECC_ucerr_alarm |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

### Boot FST frCPU check

Table 38 : Boot FST frCPU check function

| Service name: | boot_fst_frcpu_check |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_frcpu_check( void); |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | frCPU check successful. |
|  | STATUS_FIRMWARE |  | frCPU check failed. |
| Description: | This API performs the frCPU tests. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful frCPU check, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Calls the fst_common_mul_div_check () function to perform multiplier and divider check. - Calls the fst_common_bus_matrix_check () function to perform bus matrix check. - Calls the boot_fst_nvic_int_prio_check() function to perform NVIC interrupt priority check. - Calls the fst_common_mpu_nvic_reg_read() function to read the MPU and NVIC registers. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 18: Boot FST frCPU check function flow

### NVIC interrupt priority register check

Table 39 : frCPU NVIC interrupt priority register check function

| Service name: | boot_fst_nvic_int_prio_check |  |  |
| --- | --- | --- | --- |
| Syntax: | static void boot_fst_nvic_int_prio_check (void); |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | Void |  |  |
| Description: | This API performs the NVIC interrupt priority register check. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful frCPU check, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. In a for loop (for 8 interrupt priority registers), read all the used priority values from ARM Cortex-M3 NVIC priority registers, bit-wise XORing them. This is the current parity and store it. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 850153 |  |  |

Figure 19: NVIC interrupt priority register check flow

### Async commands register

Table 40 : Async commands register function

| Service name: | boot_fst_async_cmd_reg |  |  |
| --- | --- | --- | --- |
| Syntax: | Static uint32_t boot_fst_async_cmd_reg (void); |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful registrations. |
|  | Return value of register_callback_cmd() API. |  |  |
| Description: | This function registers boot FST async commands with FSTM async interface layer. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful registration, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - Register BIOS boot complete callback with FSTM async interface. With interface number = PCIE_FSTM_INTERFACE, command = 0x206U, fst id = BOOT_FST_ID and callback = boot_fst_bios_boot_cb. - Register POSC results callback with FSTM async interface. With interface number = PCIE_FSTM_INTERFACE, command = 0x218U, fst id = BOOT_FST_ID and callback = boot_fst_posc_results_cb. - Register STL results callback with FSTM async interface. With interface number = PCIE_FSTM_INTERFACE, command = 0x23EU, fst id = BOOT_FST_ID and callback = boot_fst_stl_results_cb. - Register ODCC dummy snapshot callback with FSTM async interface. With interface number = PCIE_FSTM_INTERFACE, command = 0x039EU, fst id = BOOT_FST_ID and callback = boot_fst_odcc_ss_cb. - Register override config parameter callback with FSTM async interface. With interface number = PCIE_FSTM_INTERFACE, command = 0x4D0U, fst id = BOOT_FST_ID and callback = boot_fst_override_config_param_cb. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 20 : Async commands register function flow

### Async commands de-register

Table 41 : Async commands de-register function

| Service name: | boot_fst_async_cmd_de_reg |  |  |
| --- | --- | --- | --- |
| Syntax: | Static uint32_t boot_fst_async_cmd_de_reg (void); |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful registrations. |
|  | Return value of de_register_callback_cmd () API. |  |  |
| Description: | This function de-registers boot FST async commands with FSTM async interface layer. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful de-registration, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - De-register BIOS boot complete callback with FSTM async interface. With interface number = PCIE_FSTM_INTERFACE, fst id = BOOT_FST_ID. - De-register POSC results callback with FSTM async interface. With interface number = PCIE_FSTM_INTERFACE, fst id = BOOT_FST_ID. - De-register STL results callback with FSTM async interface. With interface number = PCIE_FSTM_INTERFACE, fst id = BOOT_FST_ID. - De-register ODCC dummy snapshot callback with FSTM async interface. With interface number = PCIE_FSTM_INTERFACE, fst id = BOOT_FST_ID. - De-register override config parameter callback with FSTM async interface. With interface number = PCIE_FSTM_INTERFACE, fst id = BOOT_FST_ID. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 21 : Async commands de-register function flow

### Host boot monitoring

The asynchronous data received from host to SCI FSTs is routed through FSTM. Refer SCI SAS and FSTM MDS for more details.

Table 42 : Host monitoring application thread function

| Service name: | boot_fst_host_monitor |  |  |
| --- | --- | --- | --- |
| Syntax: | Static uint32_t boot_fst_host_monitor (void); |  |  |
| Sync/Async: | Asynchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful host monitor. |
|  | STATUS_FIRMWARE/STATUS_THREADX |  | On host monitor failure. |
| Description: | This function calls the host monitoring APIs. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful host monitoring, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | Initially, host_monitor_timeout is set to BIOS boot timeout value. This function performs the following operations. - Calls boot_fst_async_cmd_reg to register async commands. - Calls fst_lib_timer_create() to create host monitor timer. - Calls fst_lib_timer_start() to start host monitor timer. - Calls tx_event_flags_get(BOOT_FST_TIMER_EXPIRED ¦ BOOT_FST_BIOS_BOOT_COM_MSG_RECEIVED ¦ BOOT_FST_POSC_RESULTS_RECEIVED ¦ BOOT_FST_STL_RESULTS_RECEIVED ¦ BOOT_FST_ODCC_SS_RECEIVED). - If event flag BOOT_FST_BIOS_BOOT_COM_MSG_RECEIVED is set, calls the boot_fst_bios_boot_monitor() function to check the host BIOS boot status. On success, set the bios_boot_completed to 1 and host_monitor_timeout to posc results timeout. - If event flag BOOT_FST_POSC_RESULTS_RECEIVED is set and bios_boot_completed equal to 1, calls boot_fst_posc_results_monitor() function to check the POSC test results. On Success set posc_res_rec_completed to 1 and host_monitor_timeout to stl results timeout. - If event flag BOOT_FST_ODCC_SS_RECEIVED is set, calls boot_fst_odcc_check() function to comapre the ODCC dummy snapshot test results. - If event flag BOOT_FST_STL_RESULTS_RECEIVED is set and posc_res_rec_completed is equal to 1, calls boot_fst_stl_results_monitor () function to check the startup STL results. On status and response code equal to success, if host_boot_monitor_completed equal to zero set host_boot_monitor_completed to 1. - If event flag BOOT_FST_TIMER_EXPIRED is set, then update the status with timeout error. - Calls fst_lib_timer_stop() to stop the host monitor timer. On success, calls fst_lib_timeout_change() to configure host_monitor_timeout. - Repeat from step 6 to 13 in while loop, until status equal to success and host_boot_monitor_completed equal to 0. - If status is success, calls boot_fst_ravdm_check() function to test SCI communication with PUNIT. And calls boot_fst_pmc_ipc_check() function to test SCI communication with PMC. - Calls boot_fst_async_cmd_de_reg() API to de-register async commands. - If timer status is success, calls fst_lib_timer_delete() to delete host monitor timer. - Return with status. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 590419, 590782 |  |  |

Figure 22: Host monitoring application thread function flow

### Host BIOS boot monitor

Table 43 : Host BIOS boot monitor function

| Service name: | boot_fst_bios_boot_monitor |  |  |
| --- | --- | --- | --- |
| Syntax: | Static uint32_t boot_fst_bios_boot_monitor (void) |  |  |
| Sync/Async: | Asynchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful BIOS boot monitor. |
|  | STATUS_FIRMWARE |  | On BIOS boot monitor failure. |
| Description: | This API performs the BIOS boot monitor. Host BIOS on its boot completion sends a message to SCI via PCIe MB and SCI send acknowledgement to host. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful host BIOS boot monitor, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | On receiving BIOS boot complete message, boot_fst_bios_boot_cb is called. It calls tx_queue_send(boot_fst_queue) and set the event flag to notify boot FST thread waiting for messag This function performs the following operations. - Calls tx_queue_receive() to receive the BIOS boot complete message - Checks if bios_boot_msg->length equal to BOOT_FST_BIOS_BOOT_REC_LEN, if not update the response status as invalid length. - Checks if bios_boot_msg->isi_bdf_valid is equal to 0XAAu, if not update the response code status with BDF mismatch. - Update the response packet and calls tl_respond() to send the Acknowledgement to host with response code. If any error in ack then host will send the BIOS boot complete message again. - If any error in above steps report to FSTM. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 971267, 971269, 971270, 850143, 850153 |  |  |

The sequence diagram for host BIOS boot monitor is shown below.

Figure 23: Host BIOS boot monitor overall flow diagram

### POSC results packet validate

Table 44 : POSC results packet validate function

| Service name: | boot_fst_posc_results_pkt_validate |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_posc_results_pkt_validate( const struct boot_fst_posc_res_msg *result_msg, uint16_t *res_code_ptr ) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | * result_msg | struct boot_fst_posc_res_msg | Pointer to POSC results message. |
|  | *res_code_ptr | Uint16_t | Pointer to response code. |
| Parameters (out): | *res_code_ptr | Uint16_t | Pointer to response code. |
| Return value: | STATUS_SUCCESS |  | On successful POSC packet validation. |
|  | STATUS_FIRMWARE(M_FST_BOOT, E_IN_POSC_RES_MONITOR, E_INVALID_SSEAVX_DATA_VALID_FIELD) |  | If invalid AVX validate data is received.. |
|  | STATUS_FIRMWARE( M_FST_BOOT, E_IN_POSC_RES_MONITOR, E_POSC_RES_FAIL) |  | If POSC results is received is fail. |
| Description: | This API validates the POSC results message and update the status and response code if any error.. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful validation, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - Validate POSC results length, if mismatch then update response code. - Validate avx_data_valid field, if value is not INVALID then update status and response code. - Validate posc result field, if result is fail then update the status and response code. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 24 : POSC results packet validate function flow

### POSC – get number of cores enabled

Table 45 : POSC – get number of cores enabled function

| Service name: | boot_fst_num_cores_enabled_get |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint16_t boot_fst_num_cores_enabled_get( uint32_t core_mask ) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | core_mask | Uint32_t | Core mask value received in POSC results message. |
| Parameters (out): | void |  |  |
| Return value: | BOOT_FST_PCIE_SUCCESS |  | On success. |
|  | BOOT_FST_PCIE_UNKNOWN_CORE_MASK |  | Response code on failure. |
| Description: | This API will parse the 32 bits core mask(antivalent bits format) value to get the cores enabled. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful parsing, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - In a for loop of 16, parse the core mask value received (2 bit antivalent value) if core mask corresponding to core is enabled then increment the boot_fst_num_stl_res_expected. - If parsed values is not equal to enable or disable then update the response code with BOOT_FST_PCIE_UNKNOWN_CORE_MASK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 25 : POSC- get number of cores function flow

### POSC results respond

Table 46 : POSC results respond function

| Service name: | boot_fst_posc_results_respond |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_posc_results_respond( struct boot_fst_pcie_res_s *posc_res, uint32_t res_len, uint16_t response_code) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | * result_msg | struct boot_fst_posc_res_msg | Pointer to POSC results message. |
|  | res_len | uint32_t | POSC results response length |
|  | response_code | uint16_t | Response code. |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful sending response. |
|  | Return value of tl_respond |  |  |
| Description: | This API updates the POSC results response fields and respond to POSC results message. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful respond, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - Updates the flags, length and response code POSC response fileds. - Calls tl_respond to send response to POSC results message. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

### POSC test results monitor

Table 47 : POSC test results monitor function

| Service name: | boot_fst_posc_results_monitor |  |  |
| --- | --- | --- | --- |
| Syntax: | Static uint32_t boot_fst_posc_results_monitor (void) |  |  |
| Sync/Async: | Asynchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful host STL manager startup monitor. |
|  | STATUS_FIRMWARE |  | On host STL manager startup monitor failure. |
| Description: | This API performs the POSC test results monitor. |  |  |
| Preconditions: | Host BIOS boot complete message received and ack sent successfully. |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful POSC results monitor, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | On receiving POSC results message, boot_fst_ posc_results_cb is called. It calls the tx_queue_send(boot_fst_queue) and set the event flag to notify boot FST thread waiting for message. This function performs the following operations. - Calls tx_queue_receive() to receive the POSC results message. - Calls boot_fst_posc_results_pkt_validate() to validate POSC results message. - Calls boot_fst_num_cores_enabled_get() API to get number of cores enabled. - Copy the POSC results message header into posc results response header and calls boot_fst_posc_results_respond() API to respond to POSC results message.. - If any error in above steps calls errmgt_firmware_internal_error() to assert NOK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 971267, 971269, 971270, 850143, 850157, 850153 |  |  |

The sequence diagram for POSC results monitor is shown below.

Figure 26: Host POSC results monitor flow diagram

### RAVDM test

To test the RAVDM, perform RAVDM ping test and fetch the ITD parameters from PUINT.

Table 48 : RAVDM check function

| Service name: | boot_fst_ravdm_check |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_ravdm_check (void) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful RAVDM test. |
|  | STATUS_FIRMWARE |  | On RAVDM check failure. |
| Description: | This API send the ping command to PUNIT and fetch the ITD parameters from PUINT via RAVDM. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful RAVDM test, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - Calls boot_fst_ravdm_ping_check() to perform RAVDM ping test. - Calls boot_fst_ravdm_itd_fetch() to fetch the ITD parameters from PUINT via RAVDM. - If any error in above steps, return error. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 971267, 971269, 971270, 590363, 850153 |  |  |

Figure 27: RAVDM check function flow

### RAVDM ping test

Table 49 : RAVDM ping test function

| Service name: | boot_fst_ravdm_ping_check |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_ravdm_ping_check (void) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful RAVDM ping test. |
|  | STATUS_FIRMWARE |  | On RAVDM ping test failure. |
| Description: | This API sends the ping command and receives the response to/from PUNIT via RAVDM. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful RAVDM ping test, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - Prepares the ping message with PING command. - Calls tl_send(PUNIT, ping message, timeout) to send the ping message to PUINT via RAVDM. - Calls tl_receive(PUNIT, ping message, timeout) to receive the ping response from PUINT. - Check the response code, if error response code, return error. And check number of DWORDS if not ,matching return error. - If any error in above steps, return error. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 850155 |  |  |

Figure 28: RAVDM ping test function flow

### RAVDM ITD parameters fetch

Table 50 : Fetch ITD parameters function

| Service name: | boot_fst_ravdm_itd_fetch |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_ravdm_itd_fetch (void) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful fectching ITD parameters. |
|  | STATUS_FIRMWARE |  | On ITD parameters fetching failure. |
| Description: | This API sends the get ITD parametes command and receives the response to/from PUNIT via RAVDM. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful fetching the ITD parameters, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - Prepares the get ITD parameters message with get ITD parametsrs command and domain. - Calls tl_send(PUNIT, message, timeout) to send the get ITD parameter message to PUINT via RAVDM. - Calls tl_receive(PUNIT, ping message, timeout) to receive the response from PUINT. - Check the response code, if error response code, return error. And check number of DWORDS if not ,matching return error. - Copy the ITD parameters into ITD values global variable which will be shared with periodic voltage monitoring FST. - If any error in above steps, return error. - Repeate the above steps for all the domains (VCCCORE, VCCL2, VCCRING and VCCSA). |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 971282, 971283, 971284, 850155 |  |  |

Figure 29: Fetch ITD parameters function flow

### PMC IPC test

Table 51 : PMC IPC check function

| Service name: | boot_fst_pmc_ipc_check |  |  |
| --- | --- | --- | --- |
| Syntax: | Static uint32_t boot_fst_pmc_ipc_check (void) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful PMC IPC test. |
|  | STATUS_FIRMWARE |  | On PMC IPC test failure. |
| Description: | This API performs the PMC IPC test by sending platform transition information query and receiving the respose from PMC via IPC. TODO: Check if opcode validation is required, under discussion. Current driver implementation does not pass opcode to application. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful PMC IPC test, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Updates the PMC IPC test monitoring timer structure with timeout value, reload type and callback function. - Calls the fst_lib_timer_create() to create the PMC IPC test monitoring timer. - Calls the fst_lib_timer_start() to start the timer. If timer start failed then call timer delete API. - Calls the pmc_ipc_osdl_send() to send platform transition information query. Platform transition query opcode (0x0009) and send length is one. Refer PMC IPC driver MDS for more information on APIs. - Calls pmc_ipc_osdl_receive() to receive the response from PMC. - Calls fst_lib_timer_stop() to stop timer and fst_lib_timer_delete() to delete timer. Call these APIs irrespective of status from above steps. If both PMC IPC send/receive and timer stop/delete failed then return PMC IPC error. - If any error in above steps return error status. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 30: PMC IPC test flow diagram

### ODCC test – Dummy snapshot compare

While startup tests are executing if host test application sends the ODCC snapshots then compare the dummy snapshots received.

Table 52 : ODCC dummy snapshot compare

| Service name: | boot_fst_odcc_check |  |  |
| --- | --- | --- | --- |
| Syntax: | Static uint32_t boot_fst_odcc_check (void) |  |  |
| Sync/Async: | Asynchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful ODCC snapshot camparision. |
|  | Return value of odcc_dummy_ss_comparison() API. |  |  |
|  | STATUS_THREADX(M_FST_BOOT, E_IN_SNAPHSHOT_COMPARISON, tx_status) |  | On queue receive failure. |
| Description: | This API receives the two dummy snapshots from host and calls odcc_dummy_ss_comparison() API to compare the snapshots. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful host boot STLs monitor, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | On receiving ODCC snapshot message, boot_fst_odcc_ss_cb is called. It calls the tx_queue_send(boot_fst_queue) and set the event flag to notify boot FST thread waiting for message. This function performs the following operations. Calls tx_queue_receive() to receive the ODCC snapshot. If boot_fst_first_odcc_ss_received equal to zero then stop ODCC results in boot_fst_odcc_ss[0] local variable else store in boot_fst_odcc_ss[1] and call the odcc_dummy_ss_comparison(boot_fst_odcc_ss, 2). Report status to caller function. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 31: ODCC dummy snapshot compare flow diagram

### Override config param data set

Table 53 : Override config param data set function

| Service name: | boot_fst_override_data_set |  |  |
| --- | --- | --- | --- |
| Syntax: | static void boot_fst_override_data_set( const struct boot_fst_write_config_data_cmd_s *cmd_prt, struct boot_fst_write_config_data_resp_s *resp_ptr ) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | *cmd_prt | struct boot_fst_write_config_data_cmd_s | Pointer to override config param message. |
|  | *resp_ptr | struct boot_fst_write_config_data_resp_s | Pointer to override config param response message. |
| Parameters (out): | *resp_ptr | struct boot_fst_write_config_data_resp_s | Pointer to override config param response message. |
| Return value: | void |  |  |
| Description: | This API sets the configuration data and update the payload in response message. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: |  |  |  |
| Implementation comments | This function performs the following operations. If the override parameter is not equal to 0xFFFFFFFFU for any of the payload member of boot_fst_write_config_data_cmd_s, calls corresponding global config set API. The return value of the set API is updated in corresponding response message members. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 32 : Override config data set

### Override config param handle

Table 54 Override config param handle function

| Service name: | boot_fst_override_config_param |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_override_config_param(void) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): |  |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful sending response. |
|  | STATUS_THREADX(M_FST_BOOT, E_IN_CONFIG_PARSING, tx_status) |  | For queue receive failure. |
|  | Return value of isi_memcpy_s API or tl_send API. |  |  |
| Description: | This API process the override config data and sets it using glocal config APIs. And form the respons packet and responds. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful processing, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - Calls tx_queue_receive() API to receive override config data. - In response packet sets the default value for payload fields. - Calls boot_fst_override_data_set() API to set the data. - Updates the response code payload fields updated in boot_fst_override_data_set() API. - Capies the header data from received message to response message. - Calls tl_send() API to send the response to override config message. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 33 : Override config param function flow

### STL results packet validation

Table 55 : STL results packet validation function

| Service name: | boot_fst_stl_results_pkt_validate |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint16_t boot_fst_stl_results_pkt_validate( const struct boot_fst_stl_res_msg *result_msg ) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | *result_msg | struct boot_fst_stl_res_msg | Pointer to STL results message. |
| Parameters (out): | void |  |  |
| Return value: | BOOT_FST_PCIE_SUCCESS |  | On successful sending response. |
|  | BOOT_FST_PCIE_INVALID_LENGTH |  | For Invalid length |
|  | BOOT_FST_PCIE_UNKNOWN_SYS_TIME_VALID |  | For unknown system time valid field value.. |
|  | BOOT_FST_PCIE_UNKNOWN_STL_RES_VALID |  | For unknown STL results valid field value. |
|  | BOOT_FST_PCIE_UNKNOWN_STL_RES |  | For unknown STL results value. |
| Description: | This API validates the STL results message received. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful validation, else it will return response code. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - Validates length of message received. If not matching updated response code. - Validates system time validity bits, if invalid then updates response code. - Validates STL results validity bits, if invalid then updates response code. - Validates STL results bits, if unknown then updates response code. - If STL results received is valid and results is FAIL then update diagnostic data and calls errmgt_firmware_internal_error() API to assert NOK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 34 : STL results packet validation function flow

### Uncore STL execution check

Table 56 : Uncore STL execution check function

| Service name: | boot_fst_uncore_stl_exec_check |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint16_t boot_fst_uncore_stl_exec_check( uint8_t stl_core_mask) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | stl_core_mask | uint8_t | Uncore mask received. |
| Parameters (out): | void |  |  |
| Return value: | BOOT_FST_PCIE_SUCCESS |  | On successful checking the uncore mask. |
|  | BOOT_FST_PCIE_UNKNOWN_UNCORE_MASK |  | If core mask is unknown. |
| Description: | This API validates the uncore mask value received and checks if STL executed or not. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful checking uncore mask, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - If STL is executed on uncore If executed update response code to success and increment boot_fst_num_stl_res_received variable by 1. - Else if STL is not executed then update the response code to success. - Else uncore mask value is unknown so update the response code with BOOT_FST_PCIE_UNKNOWN_UNCORE_MASK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 35 : Uncore STL execution check function flow

### Core STL execution check

Table 57 : Core STL execution check function

| Service name: |  |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint16_t boot_fst_stl_exec_check( uint32_t stl_core_mask) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | stl_core_mask | Uint32_t | core mask received. |
| Parameters (out): | void |  |  |
| Return value: | BOOT_FST_PCIE_SUCCESS |  | On successful checking the core mask. |
|  | BOOT_FST_PCIE_UNKNOWN_CORE_MASK |  | If core mask is unknown. |
| Description: | This API checks the execution status of core mask. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful checking execution status, else it will return response code. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - In a for loop of 16 the below operations are executed. - Fetch the 2 bit core mask from stl_core_mask using the index. - If core mask is unknown then update the response code to BOOT_FST_PCIE_UNKNOWN_CORE_MASK. - Else if core mask STL status is executed then increment the boot_fst_num_stl_res_received by 1. - Else update the response code with success. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 36 : Core STL execution check function flow

### System time set

Table 58 : System time set function

| Service name: | boot_fst_system_time_set |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t boot_fst_system_time_set( const struct boot_fst_stl_res_msg *result_msg) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | *result_msg | struct boot_fst_stl_res_msg | Pointer to STL results message. |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful setting system time. |
|  | Return value of rtos_timer_to_sec_get |  |  |
| Description: | This API sets the system time received in STL results mesasage and also sets the RTC time in seconds. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful respond, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - In STL message if the system time is valid then calls gc_system_time_set() to set the time. - Calls rtos_timer_to_sec_get() to get the RTOS time in seconds and calls gc_rtc_time_set () to set RTC time. - If system time is invalid the calls gc_system_time_set() and gc_rtc_time_set() APIs to set time to zero. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 37 : Syatem time set function flow

### STL results respond

Table 59 : STL results respond function

| Service name: | boot_fst_stl_results_respond |  |  |
| --- | --- | --- | --- |
| Syntax: | static void boot_fst_stl_results_respond( struct boot_fst_pcie_res_s *stl_res, uint16_t response_code) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | * stl_res | struct boot_fst_stl_res_msg | Pointer to STL results message. |
|  | response_code | Uint32_t | Response code. |
| Parameters (out): | void |  |  |
| Return value: | Void. |  |  |
| Description: | This API respond to STL results message by updating the response message fields. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful respond, else it will call fw_internal_error_handler() API to assrt NOK. |  |  |
| Implementation comments | This function performs the following operations. - Update the response message fields. - Calls the tl_respond() API to respond to STL results message. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

### Host startup STL results monitor

Table 60 : Host startup STL results monitor function

| Service name: | boot_fst_stl_results_monitor |  |  |
| --- | --- | --- | --- |
| Syntax: | Static uint32_t boot_fst_stl_results_monitor (uint16_t *hbc_response_code) |  |  |
| Sync/Async: | Asynchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | *hbc_response_code | Uint16_t | Pointer to response code. |
| Parameters (out): | *hbc_response_code | Uint16_t | Pointer to response code. |
| Return value: | STATUS_SUCCESS |  | On successful host boot STLs monitor. |
|  | STATUS_FIRMWARE |  | On host boot STLs monitor failure. |
| Description: | This API performs the host boot STLs monitor. Making sure that the Host Boot STLs are run and they pass. Disassociate startup STL results from POSC Core count. If STL results is PASS then check if STL executed on at least on one core/uncore, if not assert NOK. All STL results are received only in one host boot complete message.. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful host boot STLs monitor, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | On receiving STL results(Host boot complete) message, boot_fst_stl_results_cb is called. It calls the tx_queue_send(boot_fst_queue) and set the event flag to notify boot FST thread waiting for message. This function performs the following operations. - Calls tx_queue_receive() to receive the STL results message. - Calls boot_fst_stl_results_pkt_validate() to validate packet fields. - Calls boot_fst_uncore_stl_exec_check() to check uncore execution status. - Calls boot_fst_stl_exec_check() to check core execution status for core mask 1. - Calls boot_fst_stl_exec_check() to check core execution status for core mask 2. - As part of STL results message atleast one core/uncore mask must be set else it’s a NOK condition. If boot_fst_num_stl_res_received is equal to zero then update the response code with BOOT_FST_STL_NOT_EXE_ON_ALL_CORES. Also update the status and diagnostic data to assert NOK. - Calls boot_fst_system_time_set() API to set system time. - Update the response packet and calls tl_respond() to send the Acknowledgement to host with response code. If any error in ack then host will send the STL results message again. - If any error in above steps then calls errmgt_firmware_internal_error() API to assert NOK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 971269, 971270, 850143, 850157, 850153 |  |  |

The sequence diagram for Host boot STLs run and pass test is shown below.

Figure 38: Host boot STLs monitor flow diagram

## Imported functions

### GP timer expiry callback function

| Service name: | fst_common_gp_timer_cb |  |  |
| --- | --- | --- | --- |
| Syntax: | static void fst_common_gp_timer_cb( timer_id_e id ) |  |  |
| File: | Common_checks.c |  |  |
| Sync/Async: | Asynchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | id | timer_id_e | Timer id. |
| Parameters (out): | void |  |  |
| Return value: | void |  |  |
| Description: | This API is called on GP timer expiry. |  |  |
| Preconditions: | This API must be registered while creating the GP timers. |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: |  |  |  |
| Implementation comments | On GP timer expiry, it sets the FST_COMMON_TIMER_EXPIRED event flag. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 39: GP Timer expiry callback function flow

### Periodic timer expiry callback function

| Service name: | fst_common_prd_timer_cb |  |  |
| --- | --- | --- | --- |
| Syntax: | static void fst_common_periodic_timer_cb( void ) |  |  |
| File: | Common_checks.c |  |  |
| Sync/Async: | Asynchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | void |  |  |
| Description: | This API is called on periodic timer expiry. |  |  |
| Preconditions: | This API must be registered while configuring the periodic timers. |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: |  |  |  |
| Implementation comments | On Periodic timer expiry, it sets the FST_COMMON_PERIODIC_TIMER_EXPIRED event flag. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 40 : Periodic Timer expiry callback function flow

### RTOS timer error handler

Table 61 : RTOS timer error handler function

| Service name: | fst_common_rtos_error_handler |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t fst_common_rtos_error_handler(uint8_t event_data1, uint8_t event_data2, uint8_t event_data3, rtos_timer_counter_t *rtos_test_value1, rtos_timer_counter_t *rtos_test_value2, rtos_timer_counter_t *rtos_test_value3) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | event_data1 | uint8_t | Event data 1. |
|  | event_data2 | uint8_t | Event data 2. |
|  | event_data3 | uint8_t | Event data 3. |
|  | *rtos_test_value1 | rtos_timer_counter_t | RTOS timer values. |
|  | *rtos_test_value2 | rtos_timer_counter_t | RTOS timer values. |
|  | *rtos_test_value3 | rtos_timer_counter_t | RTOS timer values. |
| Parameters (out): | void |  |  |
| Return value: | Return value of errmgt_firmware_internal_error() API. |  | If any error while asserting NOK. |
| Description: | This API updates the diagnostic data and asserts NOK for RTOS timer error. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful respond, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - Update the diagnostic data using input parameters. - Calls errmgt_firmware_internal_error() API to assert NOK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

### Timer Error Injection(EI) test

Table 62 : Timer error injection check function

| Service name: | fst_common_rtos_timer_check |  |  |
| --- | --- | --- | --- |
| Syntax: | uint32_t fst_common_timer_ei_check (void) |  |  |
| File: | boot_fst_timer_tests.c |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful timer test. |
|  | STATUS_FIRMWARE |  | On timer test failure. |
| Description: | This API performs the timer error injection test. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful RTOS timer test, else it will return errors for general errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Calls fmm_alarm_mask_unmask() to mask RTOS timer alarm. - Calls rtos_timer_osdl_ei_counters_load() function to load the error injection counters with different values. - Calls rtos_timer_osdl_ei_enable() function to enable the error injection. Soon after the enabling the error injection, free running counter NOK is set. - Calls the rtos_timer_osdl_frc_nok_sts_get() function to get the NOK status. If NOK is set, timer test is pass, else calls fst_common_rtos_error_handler() API to assert NOK. - Calls fmm_alarm_premask_status_get() to get premask status. If status is 1 then calls fst_common_rtos_error_handler() API to assert NOK. - Calls rtos_timer_osdl_ei_disable() function to disable error injection. - Calls rtos_timer_osdl_frc_reset() function to reset the free running counter. This will clear the NOK status. - Calls fmm_alarm_premask_status_get() to get the pre mask status. If status is not 1 then calls fst_common_rtos_error_handler() API to assert NOK. - Calls fmm_alarm_mask_unmask() to unmask RTOS timer alarm. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 850142, 850153 |  |  |

Figure 41: RTOS timer test flow diagram

### WDT alarm mask and wdt start

Table 63 : WDT alarm mask and wdt start function

| Service name: | fst_common_wdt_alarm_mask_and_start |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t fst_common_wdt_alarm_mask_and_start(void) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On success. |
|  | Return value of fmm_alarm_mask_unmask() or wdt_osdl_configure() or wdt_osdl_start() APIs. |  |  |
| Description: | This API mask the WDT timeout alarm, configures the WDt and starts WDT timer. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful respond, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - Calls fmm_alarm_mask_unmask() API to mask WDT alarm. - Calls wdt_osdl_configure() API to configure WDT to FST_COMMON_WDT_TIMEOUT_TICKS. - Calls wdt_osdl_start() to start WDT timer. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 42 : WDT alarm mask and wdt start function

### WDT configure and start

Table 64 : WDT configure and start function

| Service name: | fst_common_wdt_config_start |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t fst_common_wdt_config_start(void) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On success. |
|  | Return value of fmm_alarm_premask_status_get() or wdt_osdl_configure() or errmgt_firmware_internal_error() APIs. |  |  |
| Description: | This API |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful respond, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This function performs the following operations. - Calls fmm_alarm_premask_status_get() API to get premask status. - If pre-mask status is not equal to 1 then wdt_osdl_configure(WDT_DEFAULT_VALUE) and calls wdt_osdl_start() to configure and start WDT. - If pre-mask status is equal to 1 then updates the diagnostic data and calls errmgt_firmware_internal_error() API to assert NOK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 43 : WDT configure and start function flow

### WDT difference error handle

Table 65 : WDT difference error handle function

| Service name: | fst_common_wdt_diff_error_handler |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t fst_common_wdt_diff_error_handler(uint64_t rtos_timer_diff) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | rtos_timer_diff | Uint64_t | RTOS timer difference value. |
| Parameters (out): | void |  |  |
| Return value: | Return value of errmgt_firmware_internal_error(). |  |  |
| Description: | This API asserts nok for WDT if mismatch in RTOS timer difference value. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: |  |  |  |
| Implementation comments | This function performs the following operations. - Updates the diagnostic data and calls errmgt_firmware_internal_error() API. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

### WDT error handler

Table 66 : WDT error handler function

| Service name: | fst_common_wdt_error_handler |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t fst_common_wdt_error_handler(void) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): |  |  |  |
| Parameters (out): | void |  |  |
| Return value: | Return value of errmgt_firmware_internal_error() API. |  |  |
| Description: | This API asserts NOK if premask status is not equal to 1. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: |  |  |  |
| Implementation comments | This function performs the following operations. - Updates the diagnostic data and calls errmgt_firmware_internal_error() API. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

### WDT NOK status error handle

Table 67 : WDT NOK status error handle function

| Service name: | fst_common_wdt_status_nok_error |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t fst_common_wdt_status_nok_error(void) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): |  |  |  |
| Parameters (out): | void |  |  |
| Return value: | Return value of errmgt_firmware_internal_error() API. |  |  |
| Description: | This API asserts NOK if WDT NOK is not set after error injection. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: |  |  |  |
| Implementation comments | This function performs the following operations. - Updates the diagnostic data and calls errmgt_firmware_internal_error() API. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

### WDT test

Table 68 : WDT check function

| Service name: | fst_common_wdt_check |  |  |
| --- | --- | --- | --- |
| Syntax: | uint32_t fst_common_wdt_check (void) |  |  |
| File: | boot_fst_timer_tests.c |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful timer test. |
|  | STATUS_FIRMWARE |  | On timer test failure. |
| Description: | This API performs the WD timer test. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful WDT test, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Calls fst_common_wdt_alarm_mask_and_start() to mask alart and start WDT. - Calls rtos_timer_osdl_twall_clock_get() to get RTOS time stamp. - In do-while loop with retry count wait for WDT expiry, Calls wdt_osdl_nok_status_get() function to get the WDT NOK status. - If NOK is set, timer test is pass, else report as test failure by calling fst_common_wdt_status_nok_error. - Calls rtos_timer_osdl_twall_clock_get () to get RTOS time stamp. - Calculate the RTOS time stamp difference if not matching then calls fst_common_wdt_diff_error_handler() to assert NOK. - Calls fst_common_wdt_config_start() to configure WDT and start it. - Calls fmm_alarm_premask_status_get() to get premask status. If status is not equal to 1 then calls fst_common_wdt_error_handler() to assert NOK. - Calls fmm_alarm_mask_unmask() to unmask WDT timeout alarm. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 850142, 850153 |  |  |

Figure 44: WDT test flow diagram

### GP timer difference calculation

Table 69 : GP timer difference calculation function

| Service name: | fst_common_gp_timer_diff_cal |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t fst_common_gp_timer_diff_cal( rtos_timer_val_t *ts_start, timer_id_e id) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | *ts_start | rtos_timer_val_t | Pointer to start RTOS time stamp. |
|  | id | timer_id_e | Timer id |
| Parameters (out): | void |  |  |
| Return value: | Return value of rtos_timer_osdl_twall_clock_get() or errmgt_firmware_internal_error() APIs. |  |  |
| Description: | This API calculates the RTOS time stamp difference for GP timer and asserts nok if mismatch in RTOS timer difference value. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: |  |  |  |
| Implementation comments | This function performs the following operations. - Calls rtos_timer_osdl_twall_clock_get() to get stop RTOS time stamp. - Calculate the RTOS time stamp difference and convert to micro seconds and check the difference value with configured and tolerance value. - If mismatch in RTOS time difference, updates the diagnostic data and calls errmgt_firmware_internal_error() API. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 45 : GP timer difference calculation function flow

### GP timer test

Table 70 : GP timer check function

| Service name: | fst_common_gp_timer_check |  |  |
| --- | --- | --- | --- |
| Syntax: | uint32_t fst_common_gp_timer_check (void) |  |  |
| File: | boot_fst_timer_tests.c |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful timer test. |
|  | STATUS_FIRMWARE |  | On timer test failure. |
| Description: | This API performs the GP timer test. Test uses, FST timer library APIs exposed by timer service (uses GP timer driver APIs). Refer timer service, RTOS and GP timer driver MDS for details. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful GP timer test, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | To test all instances of GP timer, create and start AX_GP_TIMER_HW_NUM number of timers in a for loop. Do not delete the timer until all the timer instances are tested. Once all instances are tested then stop and delete all the used timers. This API performs the following operations - Create event flag to get notification of GP timer expiry interrupt. - In a for loop: - Updates fst_timer_t structure variable with timeout value, reload type and fst_common_gp_timer_cb callback. - Calls fst_lib_timer_create() function to create timer. This API internally updates the timer id in fst_timer_t structure variable. Use the timer id to start, stop and delete the timer. - Calls the fst_lib_timer_start(id) function to start the timer. - Calls rtos_timer_osdl_twall_clock_get(pointer to start time stamp stucture) function to get the RTOS timer time stamp. - Calls tx_event_flags_get() function to wait (do not wait forever) for FST_COMMON_TIMER_EXPIRED event flag to be set. The event flag is set in timer expiry callback function registered above. - If the FST_COMMON_TIMER_EXPIRED event flag is set successfully within the time. - Calls fst_common_gp_timer_diff_cal() to calculate RTOS timestamp difference and assert NOK if any mismatch. - In a for loop, calls fst_lib_timer_stop(id) and fst_lib_timer_delete(id) functions to stop and delete the timer used above. - Calls tx_event_flags_delete() function to delete the event flag created in first step. - If any error in above steps return error. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 850142, 850153 |  |  |

The sequence diagram for GP timer test is shown below.

Figure 46: GP timer test flow diagram

### Periodic timer difference calculation

Table 71 : Periodic timer difference calculation function

| Service name: | fst_common_prd_timer_diff_cal |  |  |
| --- | --- | --- | --- |
| Syntax: | static uint32_t fst_common_prd_timer_diff_cal(rtos_timer_val_t *ts_start, rtos_timer_val_t *ts_stop) |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | *ts_start | rtos_timer_val_t | Pointer to start time stamp. |
|  | *ts_stop | rtos_timer_val_t | Pointer to stop time stamp. |
| Parameters (out): | void |  |  |
| Return value: | Return value of errmgt_firmware_internal_error() API. |  |  |
| Description: | This API calculates the RTOS time stamp difference and assert NOK if any mismatch. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: |  |  |  |
| Implementation comments | This function performs the following operations. - Calculates the RTOS timestamp difference using input parameters. - Convert the difference time to micro seconds. - If any mismatch in RTOS time stamp difference then update the diagnostic data and calls errmgt_firmware_internal_error() API to assert NOK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | Private Function |  |  |

Figure 47 : Periodic timer difference calculation function flow

### Periodic timer test

Table 72 : Periodic timer check function

| Service name: | fst_common_prd_timer_check |  |  |
| --- | --- | --- | --- |
| Syntax: | uint32_t fst_common_periodic_timer_check (void) |  |  |
| File: | boot_fst_timer_tests.c |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On successful timer test. |
|  | STATUS_FIRMWARE |  | On timer test failure. |
| Description: | This API performs the periodic timer test. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful periodic timer test, else it will return errors. The upper layer will have to take care of error returned. |  |  |
| Implementation comments | This API performs the following operations. - Calls tx_event_flags_create() to create fst_common_event_flags. - Updates periodic_timer_config_s structure variable with timeout value and periodic timer expiry callback function. - Calls periodic_timer_osdl_configure() function to configure timeout and callback function. - Calls the periodic_timer_osdl_start() function to start the periodic timer. - Calls rtos_timer_osdl_twall_clock_get(pointer to start time stamp stucture) function to get the RTOS timer time stamp. - Calls tx_event_flags_get() function to wait (do not wait forever) for FST_COMMON_PERIODIC_TIMER_EXPIRED event flag to be set. The event flag is set in timer expiry callback function registered above. - If the FST_COMMON_PERIODIC_TIMER_EXPIRED event flag is set successfully within the time then calls rtos_timer_osdl_twall_clock_get(pointer to stop time stamp) function to get the RTOS timer time stamp. - Calls fst_common_prd_timer_diff_cal(&ts_start, &ts_stop) to handle time stamp. - Calls periodic_timer_osdl_stop() to stop periodic timer. - Calls tx_event_flags_delete() to delete event flags. - If any error in above steps return error. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 850142, 850153 |  |  |

Figure 48: Periodic timer test flow diagram

### FMM Scratchpad register test

Table 73 : FMM scratchpad register check function

| Service name: | fst_common_fmm_sp_reg_check |  |  |
| --- | --- | --- | --- |
| Syntax: | uint32_t fst_common_fmm_sp_reg_check (void) |  |  |
| File: | boot_fst_timer_tests.c |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | On FMM scratchpad register check successful. |
|  | STATUS_FIRMWARE |  | On FMM scratchpad register check failure. |
| Description: | This API performs the FMM scratchpad register check. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful FMM scratchpad register check, else it will call error handling API to assert NOK. |  |  |
| Implementation comments | It performs the following operations. - Call the fmm_scratchpad_reg_rw () function to write 0xA5A5A5A5U to FMM scratchpad register. The function returns the read back value from scratchpad register, store it in read_data_1. - Call the fmm_scratchpad_reg_rw () function to write 0x5A5A5A5AU to FMM scratchpad register. The function returns the read back value from scratchpad register, store it in read_data_2. - Compare write and read data return success if matching. If mismatch the update the diagnostic data and calls errmgt_firmware_internal_error() API to assert NOK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 850139, 850153 |  |  |

The sequence diagram for FMM scratchpad register test is shown below.

Figure 49: FMM scratchpad register check flow diagram

### Multiplier and Divider check

Table 74 : Multiplier and Divider check function

| Service name: | fst_common_mul_div_check |  |  |
| --- | --- | --- | --- |
| Syntax: | uint32_t fst_common_mul_div_check (void); |  |  |
| File: | fst_common_checks.c |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | Multiplier and divider check successful. |
|  | STATUS_FIRMWARE |  | Multiplier and divider check failed. |
| Description: | This API implements the multiplier and divider check functionality. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful multiplier and divider check, else it will call error handling API to assert NOK. |  |  |
| Implementation comments | It performs the following operations. - Multiplication in first “for loop”: - Perform the multiplication of input vector 1 and input vector 2 index elements. The result is 32 bit truncated value. - For each vector index, multiplication should be performed three times. First time, multiply positive numbers, second and third time, negating one of the two input vectors. - Compare the result with corresponding expected output vector index element. If mismatch in comparision, break the loop and return error to calling function. - Division in second “for loop”: - Perform the division of input vector 1 and input vector 2 index elements (input vectors are different for division). The result is quotient of signed division. - For each vector index, division should be performed three times. First time, divide positive numbers, second and third time, negating one of the two input vectors. - Compare the result with corresponding expected output vector index element. If mismatch in comparision, break the loop and return error to calling function. - Multiply and accumulate in third “for loop”: - Perform unsigned long long multiply and accumulate. - For each vector index, each number from first multiplication vector and first element of the second multiplication vector are multiplied and accumulated. - Compare the multiply and accumulate result with expected result. If mismatch in comparision, calls errmgt_firmware_internal_error() API to assert NOK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 850153 |  |  |

Figure 50: frCPU Multipler and Divider check flow

### Bus Matrix check

Table 75 : Bus Matrix check function

| Service name: | fst_common_bus_matrix_check |  |  |
| --- | --- | --- | --- |
| Syntax: | uint32_t fst_common_bus_matrix_check (uint8_t *mem_loc); |  |  |
| File: | fst_common_checks.c |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | *mem_loc | uint8_t | Pointer to single port or dual port memory location. |
| Parameters (out): | void |  |  |
| Return value: | STATUS_SUCCESS |  | Bus matrix check successful. |
|  | STATUS_FIRMWARE |  | Bus matrix check failed. |
| Description: | This API implements the bus matrix check functionality. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: | The function will return success on successful bus matrix check, else it will call error handling API to assert NOK. |  |  |
| Implementation comments | Note: Memory location may change. The memory locations of global variable is used for single port memory regions. The memory locations used for dual port memory regions are: - mem_loc_0 = 0x8FFF8U - mem_loc_1 = 0x8FFF9U - mem_loc_2 = 0x8FFFAU - mem_loc_3 = 0x8FFFBU - mem_loc_4 = 0x8FFFCU This API performs the following operations. - Word write-read-compare operation. - Write 0xAAAAAAAA to SRAM location mem_loc_0 and write 0x55555555 to SRAM location mem_loc_4. - Read back the SRAM location mem_loc_0 and mem_loc_4, compare with the write data. If mismatch in comparision, return error to calling function. - Aligned and unaligned accesses of variouse data size to fixed SRAM locations. - Two aligned word write to consecutive SRAM locations. - Write 0x12345678U to SRAM location mem_loc_0 and mem_loc_4. - Read three unaligned word from SRAM location mem_loc_1, mem_loc_2 and mem_loc_3. Compare with expected results, if mismatch, return error to calling function. - Read two unaligned half-word from SRAM location mem_loc_1 and mem_loc_3. Compare with expected results, if mismatch, return error to calling function. - Three unaligned word write to SRAM location. - Write 0x12345678U to SRAM locations mem_loc_1, mem_loc_2 and mem_loc_3. - Read three bytes from SRAM locations mem_loc_1, mem_loc_2 and mem_loc_3 respectively. Compare with expected results, if mismatch, return error to calling function. - Two unaligned half-word write to SRAM. - Write half-word 0x1234U to SRAM location mem_loc_1 and mem_loc_3. - Two byte write to SRAM location. - Write byte 0x12U to SRAM location mem_loc_1 and mem_loc_2. - Aligned word read and compare. - Read words from SRAM location mem_loc_0 and mem_loc_4. Compare with expected results, if mismatch, return error to calling function. If any error in above steps calls errmgt_firmware_internal_error() API to assert NOK. |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 848945, 850168, 850153 |  |  |

Figure 51: frCPU bus matrix check flow

### MPU and NVIC registers read

Table 76 : frCPU MPU and NVIC register read function

| Service name: | fst_common_mpu_nvic_reg_read |  |  |
| --- | --- | --- | --- |
| Syntax: | void fst_common_mpu_nvic_reg_read (void); |  |  |
| File: | fst_common_checks.c |  |  |
| Sync/Async: | Synchronous |  |  |
| Reentrancy: | Reentrant |  |  |
| Parameters (in): | void |  |  |
| Parameters (out): | void |  |  |
| Return value: | void |  |  |
| Description: | This API reads the MPU and NVIC registers. |  |  |
| Preconditions: |  |  |  |
| Timing Constraints: |  |  |  |
| Caveats: |  |  |  |
| Configuration: |  |  |  |
| Error Handling: |  |  |  |
| Implementation comments | This API performs the following operations. - Reads the following registers. MPU registers to read: - MPU Type Register, address 0xE000ED90U - MPU Control Register, address 0xE000ED94U - MPU Region Base Address register, address 0xE000ED9CU NVIC registers to read: - Configuration Control Register, address 0xE000ED14U - System Handler Control and State Register, address 0xE000ED24U - System Handler Priority Registers 1, address 0xE000ED18U - System Handler Priority Registers 2 lower halfword, address 0xE000ED20U - Debug Halting Control and Status Register, address 0xE000EDF0U - Debug Exception and Monitor Control Register, address 0xE000EDFCU - Vector Table Offset Register, address 0xE000ED08U - Application Interrupt and Reset Control Register, address 0xE000ED0CU |  |  |
| Design Decisions |  |  |  |
| SAS traceability | 850153 |  |  |

Figure 52: frCPU MPU and NVIC register read function flow

## Imported Types

| Syntax: | struct boot_fst_config_s |  |
| --- | --- | --- |
| Type: | struct |  |
| File: | boot_fst.h |  |
| Range: |  |  |
| Description: | Boot FST configuration parameters. |  |
| Members: | uint32_t bios_stage2_timer_val | Bios stage 2 timer time value. |

Table 77 : ThreadX errors

| Byte allocate errors | TX_SUCCESS, TX_DELETED, TX_NO_MEMORY, TX_WAIT_ABORTED, TX_POOL_ERROR, TX_PTR_ERROR, TX_SIZE_ERROR |
| --- | --- |
| Mutex create/get/put errors | TX_SUCCESS, TX_MUTEX_ERROR, TX_CALLER_ERROR, TX_INHERIT_ERROR,TX_DELETED, TX_WAIT_ERROR, TX_WAIT_ABORTED,TX_NOT_AVAILABLE,TX_NOT_OWNED |
| Thread create errors | TX_SUCCESS, TX_THREAD_ERROR, TX_PTR_ERROR, TX_SIZE_ERROR, TX_PRIORITY_ERROR, TX_THRESH_ERROR, TX_START_ERROR, TX_CALLER_ERROR |
| Queue create/receive | TX_SUCCESS, TX_QUEUE_ERROR, TX_PTR_ERROR, TX_SIZE_ERROR, TX_CALLER_ERROR, TX_DELETED, TX_QUEUE_EMPTY, TX_WAIT_ABORTED, TX_WAIT_ERROR |
| Event flags create/set/get/delete | TX_SUCCESS, TX_GROUP_ERROR, TX_CALLER_ERROR, TX_DELETED, TX_NO_EVENTS, TX_WAIT_ABORTED, TX_GROUP_ERROR, TX_PTR_ERROR, TX_WAIT_ERROR, TX_OPTION_ERROR. |

# Data Description

Table 78 : Global Data Description

| Varialbles | Type | Description |
| --- | --- | --- |
| boot_fst_bios_boot_msg | struct | PCIe MB message format for BIOS boot complete |
| boot_fst_posc_res_msg | struct | PCIe MB message format for POSC test results. |
| boot_fst_stl_res_msg | struct | PCIe MB message format for startup STL results. |
| boot_fst_pcie_res_s | struct | PCIe MB response message format |
| boot_fst_odcc_ss_msg | struct | PCIe MB message format for ODCC SnapShot(SS) |
| boot_fst_odcc_ss_res | struct | PCIe MB response message format for ODCC SnapShot(SS). |
| boot_fst_punit_response | struct | Response message from PUINT. |
| boot_fst_itd_res_data_0 | struct | ITD response data 0. |
| boot_fst_itd_res_data_1 | struct | ITD response data 1. |
| boot_fst_itd_res_data_2 | struct | ITD response data 2. |
| boot_stored_parity | uint32_t | NVIC interrupt priority register stored parity. |
| fst_bm_sp_sram_loc[8U] | Volatile uint8_t | Single Port SRAM memory location |
| fst_bm_dp_sram_loc[8U] __attribute__((at(0x8FFF8U))) | volatile uint8_t | Dual Port SRAM memory location. |

Table 79 : Macros Data Description

| Macros | Value | Discription |
| --- | --- | --- |
| BOOT_FST_SUCCESS_RES_CODE | 0x0U | Response code for success |
| BOOT_FST_UNSUP_CMD_RES_CODE | 0x1U | PUINT response code for unsupported command. |
| BOOT_FST_PCIE_SUCCESS | 0U | Boot fst pcie success |
| BOOT_FST_PCIE_POSC_RES_SUCCESS | 0U | Boot fst pcie resonse success |
| BOOT_FST_PCIE_INVALID_LENGTH | 0x600BU | Boot fst invalid length. |
| BOOT_FST_PCIE_UNKNOWN_SYS_TIME_VALID | 0x6538U |  |
| BOOT_FST_PCIE_UNKNOWN_STL_RES_VALID | 0x656FU |  |
| BOOT_FST_PCIE_UNKNOWN_STL_RES | 0x6576U |  |
| BOOT_FST_PCIE_UNKNOWN_CORE_MASK | 0x6507U | Boot fst unknown core mask |
| BOOT_FST_PCIE_UNKNOWN_UNCORE_MASK | 0x65A6U | Boot fst unknown uncore mask |
| BOOT_FST_PCIE_STL_NOT_EXECUTED_ON_ALL_CORES | 0x6C4EU | PCIE stl not executed on all cores. |
| BOOT_FST_PCIE_STL_NOT_EXECUTED_ON_EXPECTED_CORE_UNCORE | 0x651EU | PCIE stl not executed on expected core and uncore. |
| BOOT_FST_BB_UNKNOWN_BDF_FIELD | 0x6E7EU | Boot fst BDF mismatch. |
| BOOT_FST_ODCC_COMPARE_PASS | 0x55U | ODCC compare passed. |
| BOOT_FST_ODCC_COMPARE_FAILED | 0xAAU | ODCC compare failed. |
| BOOT_FST_ODCC_FIRST_SS_RECEIVED | 0xA5U | ODCC first snap shot received. |
| BOOT_FST_UNCORE_STL_EXEC | 0x1U | Boot fst uncore stl execution. |
| BOOT_FST_UNCORE_STL_NOT_EXEC | 0x2U | Boot fst uncore stl non execution. |
| BOOT_FST_CORE_STL_EXEC | 0x1U | Boot fst core stl execution. |
| BOOT_FST_CORE_STL_NOT_EXEC | 0x2U | Boot fst core stl non execution. |
| BOOT_FST_CM1_SW | 5868530U | Clock monitor 1 sample window |
| BOOT_FST_CM1_TC | 5000U | Clock monitor 1 target count |
| BOOT_FST_CM1_CE | 10U | Clock monitor 1 Count Error |
| BOOT_FST_CM2_SW | 49U | Clock monitor 2 sample window |
| BOOT_FST_CM2_TC | 254U | Clock monitor 2 target count |
| BOOT_FST_CM2_CE | 30U | Clock monitor 2 Count Error |
| BOOT_FST_CM3_SW | 130U | Clock monitor 3 sample window |
| BOOT_FST_CM3_TC | 25U | Clock monitor 3 target count |
| BOOT_FST_CM3_CE | 27U | Clock monitor 3 Count Error |
| NUM_CLK_MONITOR | 3U | Number of clock monitors. |
| BOOT_FST_CRC_CHECK_FAIL_RES_CODE | 0X2U | PUINT response code for CRC check failure. |
| BOOT_FST_TIMEOUT_RES_CODE | 0X3U | PUINT response code for timeout during execution. |
| BOOT_FST_NUM_ARMCM3_INT_PRIO_REG | 0X8U | Number of ARM CM3 NVIC interrupt priority registers. |
| BOOT_FST_NUM_PRIO_PER_REG | 0x4U | Number of priority values per register. |
| BOOT_FST_WIDTH_PRIO_IN_REG | 0x8U | Width of priority in register |
| BOOT_FST_PTI_QUERY_OPCODE | 0x0009U | Platform Transition Information query opcode. |
| BOOT_FST_PMC_IPC_TIMER_TIMEOUT | 200000U | PMC IPC communication monitoring timer value. |
| BOOT_FST_BIOS_BOOT_COM_CMD | 0x206U | Command for BIOS boot complete message. |
| BOOT_FST_POSC_RES_CMD | 0x218U | Command for POSC test results message. |
| BOOT_FST_HOST_BOOT_COM_CMD | 0x21FU | Command for host boot complete |
| BOOT_FST_HOST_ODCC_SS_RES_CMD | 0x6A8U | Command for ODCC shapshot response. |
| BOOT_FST_BIOS_BOOT_REC_LEN | 0x4U | BIOS boot received message length in Dwords. |
| BOOT_FST_BIOS_BOOT_RES_LEN | 0x4U | BIOS boot response message length in Dwords. |
| BOOT_FST_POSC_REC_LEN | 0xFU | POSC results received message length in Dwords. |
| BOOT_FST_POSC_RES_LEN | 0x4U | POSC results response message length in Dwords. |
| BOOT_FST_ODCC_REC_LEN | 0x6U | ODCC snapshot received message length in Dwords. |
| BOOT_FST_ODCC_RES_LEN | 0x4U | ODCC snapshot response message length in Dwords. |
| BOOT_FST_STL_REC_LEN | 0x7U | STL results received message length in Dwords. |
| BOOT_FST_RES_FLAG | 0X1U | Response flag value. |
| BOOT_FST_PCIE_STL_RES_PASS | 0x55U | STL results pass value. |
| BOOT_FST_PCIE_STL_RES_FAIL | 0xAAU | STL results fail value. |
| BOOT_FST_PCIE_RES_VALID | 0x55U | System time or STL results valid. |
| BOOT_FST_PCIE_RES_INVALID | 0xAAU | ystem time or STL results invalid. |
| BOOT_FST_CONST_FFFF | 0xFFFFU | Macro for constant value. |
| BOOT_FST_BYTE_PER_DWORD | 0x4U | Number of bytes per Dword. |
| FST_COMMON_PRD_TIME_DIFF_TOLERANCE | 30 | Periodic timer timestamp difference tolerance. |
| FST_COMMON_GP_TIME_DIFF_TOLERANCE | 65 | GP timer timestamp difference tolerance. |
| FST_COMMON_WDT_TIME_DIFF_TOLERANCE | 30 | Watchdog timer timestamp difference tolerance. |
| FST_COMMON_WDT_TIME_LOW_TOLERANCE | 1 | Watchdog timer timestamp difference lower tolerance. |
| FST_COMMON_GP_TIMER_EXPIRED | 0x1 | GP timer expiry event flags value. |
| FST_COMMON_PRD_TIMER_EXPIRED | 0x2 | Periodic timer expiry event flags value. |
| FST_COMMON_GP_TIMER_TIMEOUT | 200 | GP timer timeout in micro seconds. |
| FST_COMMON_PRD_TIMER_TIMEOUT | 250 | Periodic timer timeout in micro seconds. |
| FST_COMMON_EVENT_FLAG_TIMEOUT | 2000 | Event flags timeout. |
| WDT_DEFAULT_VALUE | 0xFFFFFFFF | Default value of WDT. |
| FST_COMMON_WDT_TIMEOUT_TICKS | 20000 | WDT timeout in ticks. If F = 200Mhz, for 100usec timeout ticks are equal to 20,000. |
| FST_COMMON_WDT_TIMEOUT_MICRO_SEC | 100 | WDT timeout in micro seconds. Note: If ticks are changed then change this value. |
| FST_COMMON_WDT_RETRY_COUNT | 100000 | WDT error status get retry count. |